# Power Operations Bulletin #124

ERCOT has revised the Day Ahead Desk section of the Operating Procedure Manual.

Changes include updating section 2.18 – added new step four.

| ## 2.18 Extend, Adjust or Suspend the Day-Ahead Scheduling Process |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | IF the Day-Ahead scheduling process is to be extended to a two Day-Ahead process, THEN ADJUST the process accordingly AND GO TO Step 5. |
| 2 | IF the Day-Ahead scheduling process timing is to be changed, THEN ADJUST the timing accordingly AND GO TO Step 5. |
| 3 | IF the Day-Ahead Market software fails without expectation of timely repair or some emergency event (Black Start, etc.) makes market operations impossible THEN: - SUSPEND market operations by emergency notification and issue a hotline call. This call will include - Nature of emergency - Interim instructions - Estimated duration - NOTIFY all positions that market operations have been suspended. - REQUEST the Transmission & Security Desk to issue a Hotline call and an Emergency Notification as follows: On the Emergency Notification System: “EMG NOTICE-DA MKT Failed-see MIS” In the MIS and a Hotline call: “The ERCOT market software has failed and is not expected to function in time to clear the market for <MM/DD/YY>. ERCOT therefore declares a market emergency and directs all QSEs to maintain Ancillary Services obligations as detailed for <PREVIOUS DAY> in the coming market period.” - GO TO Step 6 |
| 4 | If the clearing of the Day Ahead Ancillary Services market fails THEN: 4.1 Contact EMMS Production Support 4.2 Issue a hotline call to all QSEs with the following message: “ERCOT Ancillary Services Day Ahead market failed to clear. The market will be extended until further notice. Do not resubmit bids or make changes until further notice.” 4.3 Go to Step 5 |
| 5 | REQUEST that the Transmission & Security Desk Operator issue an Emergency Notification - INFORM all other desks and Shift Supervisor of the Emergency Notification. |
| 6 | WHEN the Emergency no longer exists, THEN REQUEST the Transmission/Security Desk to terminate the Emergency Notification. |

A copy of the updated procedure, Day Ahead Desk V3R15, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.