# ERCOT Operations Bulletin 230

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

Changes include incorporating new section 2.4.2.

| ### 2.4.2 Loss of Unit/Resource Plan Update |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | After the loss of a resource, a QSE has the ability to update their resource plan. If the resource plan is updated after the Operating Period Desk Operator has run the Hour Ahead study, real time balancing results could be incorrect. |
| 1 | When a QSE has the loss of any resource greater than 200 MW, contact that QSE and request what interval ending was the first interval they were able to update their resource plan. (Resource Plans can be updated for interval ending, every 15 minutes) |
| 2 | Supply the Operating Period Desk Operator with: - Name of the QSE - Resource name - MW amount of change - First interval ending that was updated |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.