# ERCOT Operations Bulletin 239

ERCOT has revised the Operating Period Desk section of the Operating Procedure Manual.

Changes include updating Appendix 4 “How to Deploy NSRS” and adding new Appendix 8 “Correcting a Large Delta Caused by NSRS Deployment.”

## Appendix 4:  How to Deploy NSRS

DEPLOY NSRS

Reference Displays:  Non-Spin Deployment display in MOI.

NOTE:  NSRS MUST be deployed for at least one hour.

To open the Non-Spin window in the ERCOT Market Operator Interface (MOI), the operator may:

    - SELECT WINDOW on control bar at the top of the MOI and select the “Non-Spin Deployment” from the scroll down menu

OR

    - If non-spin window has already been opened, operator may maximize the window located inside the MOI

SELECT the appropriate date:

    - RIGHT CLICK in the DATE box.  POP-UP calendar will appear, LEFT CLICK the desired date and SELECT “OK” to accept date or CANCEL to close POP-UP box

OR

    - Use the up and down arrow keys to select desired date.

CLICK on the GREEN PLUS (+) on the tool bar at top of page, which will add a blank row.

RIGHT CLICK on “Zone” field and enter the appropriate zone, start/end date and time, and comments for that interval.  Save/recall NSRS by pressing F10.

## Appendix 8:  Correcting a Large Delta Caused by NSRS Deployment

To correct for a large delta which was caused by the deployment of NSRS, the operators may do the following:

- Rerun the Hour Ahead Study

- Update the generation value in the Offset Calculation Spreadsheet with values derived from the newest study.

NOTE:  Step two is only performed if the generation value is different when a new study has been conducted and an updated value was given.

- Add the amount of NSRS deployed into the energy column in the Offset Calculation Spreadsheet.

- Continue with calculation of offset.

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.