# ERCOT Operations Bulletin 247

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

Changes include incorporating new section 2.5.9.

| ### 2.5.9 Scheduling Emergency Power Across the South (Eagle Pass) DC Tie |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | Declare the appropriate emergency notification. |
| 2 | ERCOT will contact AEP TDSP to verify if power is available for transfer on the Eagle Pass DC Tie. If power is available for transfer, determine the MW amount, how soon it would be available, and duration of the schedule. |
| 3 | ERCOT will contact AEP QSE to confirm the emergency schedule. |
| 4 | ERCOT must give AEP QSE a VDI and notify the ERCOT settlements department. |
| 5 | Approximately 20 minutes before the schedule begins, ERCOT will contact AEP TDSP to verify that AEP will start the emergency schedule across the Eagle Pass DC tie. ERCOT should verify the start time, MW level, and duration of the emergency schedule. |
| 6 | Approximately 20 minutes before the schedule ends, ERCOT will contact AEP TDSP to verify the stop time of the emergency schedule across the Eagle Pass DC tie. |

A copy of the bulletin is available at http://www.ercot.com/services/comm/mkt_notices/pobs/index.html.