# ERCOT Operations Bulletin 250

ERCOT has revised the Shift Supervisor Desk section of the Operating Procedure Manual.

Changes include updating section 2.2.10 – first Note and step 1; renaming Attachment B to ‘Event Investigation Form – ERCOT Compliance.’

| ### ERCOT Compliance – Internal Reporting |  |
| --- | --- |
|  | ###### Procedural Steps |
| NOTE: | There are certain events listed in the Event Investigation Process that requires an e-mail from System Operations to ERCOT Operations Support and Compliance. An initial report from System Operations will trigger the investigation process. The subject line of the e-mail should read “ERCOT Compliance – Internal Reporting.” |
| NOTE: | Events that are deemed to be significant are listed but not limited to the following: - EECP Implementation - Emergency Notice issued - Simultaneous loss of any combination of three generating unit facilities, transmission lines, and autotransformers (138KV and above). Note: Count combined cycle trains as one unit. - SPS activation - RAP implementation requiring load shedding - Under frequency relay operations (or frequency deviations greater than .7Hz from scheduled frequency). - Sustained voltage deviations greater than five percent on 345 Kv systems. - Alerts for significant transmission elements (345 Kv). - Failure of computer systems at ERCOT, the Qualified Scheduling Entity (QSE) or Transmission Operator (TO). - Voltage collapse on portions of 138 Kv or 345 Kv system - Loss of an ERCOT, QSE, or TO control center. - Black Start initiation - Uncontrollable loss of 150 MW or more of firm load shed for more than 15 minutes from a single incident. |
| NOTE: | There are also DOE and several NERC reportable events that could trigger an investigation. To report such an event, see section 2.2.11 |
| 1 | When a significant event occurs, within one hour complete the form located in Attachment B “Event Investigation Form – ERCOT Compliance” and email to: James Hinson, Steve Myers, Kent Saathoff, Kent Grammer, John Dumas, Leo Villanueva, Jeff Healy, and ERCOT Compliance Team (1 ERCOT Compliance Team). |

## Attachment B:  Event Investigation Form – ERCOT Compliance

  - Operator

  - Event

Brief Explanation of what occurred:

  - Date/Time  _______________________________________________________

  - Location  ________________________________________________________

  - Contact Information

  - Company _______________________________________________________

  - Person

  - Was this event DOE or NERC Reportable?  Yes   	No

A copy of the procedure is available at http://www.ercot.com/services/comm/mkt_notices/pobs/index.html.