# ERCOT Operations Bulletin 255

ERCOT has revised the Shift Supervisor Desk section of the Operating Procedure Manual.

Changes include adding step 3a to section 2.2.2.

| ### Capacity Shortage |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | If the load forecast is reasonable AND the “Max Capacity Room” on the “Market Analyst Interface” (MAI) shows insufficient capacity, ISSUE an OCN requesting the QSEs to update their Resource Plans, specifically for the time frame which shows the capacity insufficiency. |
| 2 | After a brief time (not to exceed two hours), if the MAI still shows a capacity shortage, ISSUE an Alert stating that the Maximum Capability of all units shown in the Resource Plans is less than the load forecast for the hours in question. |
| 3a | Refer to Day Ahead Desk Procedure for Adjustment Period RPRS market (currently section 2.15) and if sufficient time is available, perform that step, otherwise continue. |
| 3b | OOMC enough generation online to correct the predicted capacity shortage. Apply the following guidelines: - Using the “Lead Time” in the Resource Plan, determine which units are available for the time frame in which the capacity shortage is predicted. - Once the available unit (s) have been identified, choose the unit (s) that will maintain the system in it’s most secure state by considering: - Local congestion - Zonal congestion - Reactive reserves for areas with voltage concerns - Unit Reliability If necessary, direct the Day Ahead Operator to run a study and determine which units will relieve local and/or zonal congestion most effectively. |
| 4 | Enter the following information into the log: - The unit(s) that received the OOMCs - The reason each unit was chosen - The duration of the OOMC instruction |
| 5 | When the MAI shows sufficient generation capacity to cover the load forecast, discontinue the Alert. |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.