# ERCOT Operations Bulletin 265

ERCOT has revised the Day Ahead Desk Procedure.  Revision includes incorporating new section 2.6 and updated section 2.15.

| ## Notify Market of Capacity Insufficiency Identified After Schedule Matching ## at 1115 |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | IDENTIFY capacity insufficiencies by looking at the following web page: http://mospublic.ercot.com/ercot/jsp/sufficiency_report.jsp |
| 2 | Identify CSC Congestion based on an evaluation of the QSE data at the following link: http://mospublic.ercot.com/ercot/jsp/congestion_notification.jsp |
| 3 | If capacity insufficiencies or CSC congestion exist for any interval, POST the following message using the Emergency Notification System. - Post on ENS: “CapInsuf or CSC Cong (mmddyy)” |

| ## Initiate ERCOT Adjustment Period Replacement MARKET |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | - If either the MAI or the results of section 2.14 indicates that an additional Replacement MARKET is needed, DISCUSS with the Shift Supervisor. - If it is determined that a second Market is needed, CONTINUE. |
| NOTE: | - In the following steps, “X” designates the time at which the need for additional RPRS is identified (adjusted to the top of the next clock hour). |
| 1 | - At least 10 minutes prior to time X, but no less than two hours prior to the start of the first Operating Hour that the additional Replacement Reserve Services are required, OPEN a RPRS market by selecting the PUBLISH button on the Study page for the appropriate hours through hour ending 2400 of the day of the study. |
| 2 | AT time X, ISSUE an ALERT including a statement that says: - Post on MIS: “Conditions have developed such that additional Replacement Reserve Services are needed and at <top of clock hour> ERCOT opened an Adjustment Period Replacement market for the remainder of the day. QSE’s have 30 minutes to update their schedules before ERCOT purchases additional Replacement Reserves.” - Post on ENS: “ALERT – RPRS mkt opened – see MIS” - Issue a Hotline call to QSEs with a message similar to: “Additional Replacement Reserve Services are needed. An additional Replacement Reserve Services market has been opened. Refer to Alert message for details.” - Generic constraints used in the STUDY that identified the need for the MARKET should remain unchanged. - Request support from Operations Engineer as needed. |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.