# ERCOT Operations Bulletin 267

ERCOT has revised the Frequency Control Desk Procedure.  Revision includes updating section 2.8.1 and 2.8.3.

| ### Approve NERC Tags for DC Ties |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| PURPOSE: | A NERC Tag received in the OATI regarding DC Tie flows into or out of ERCOT must be reviewed for accuracy, and should be in agreement with the Market Operating System (MOS) except for changes made during the Operating Period such as curtailments. |
| 1 | When a new or updated tag is received in the OATI system, compare the tag data to the schedule in the MOS. - If the tag is greater than the MOI schedule, then the MOI schedule does not have enough generation to cover the tag. Deny the tag. - If the tag is less than or equal to the MOI schedule, which means there is enough generation scheduled, then continue to Step 2. |
| NOTE: | Data from the OATI tag may be entered into the Operations Spreadsheet at this time for purposes of comparison. |

| ### 2.8.3 Managing and Documenting Daily Inadvertent and Payback South DC Tie |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| OVERVIEW: | Inadvertent Energy flows across the DC Tie need to be monitored frequently enough during the day to ensure that large Inadvertent Energy flows don’t accumulate. |
| NOTE: | Payback will only be scheduled when there is room on the tie. Power schedules of market participants will supersede any inadvertent payback schedule. |
| NOTE: | Normally no MWs will be scheduled on the DC Tie. All the power that flows will be treated as inadvertent. |
| 1 | After Midnight: - Determine if the daily “Total Actual Flow” values on the spreadsheet agree with the AEP Corpus Christi TDSP total. If, not, then correct the spreadsheet as necessary. - Verify the values in the “Inadvertent on DC Tie” and “Inadvertent Payback” columns. Correct as necessary. |
| 2 | Verify planned flows for the next day with the AEP Corpus Christi TDSP Operator. |
| 3 | Transfer the current day’s “Inadvertent Account Total” to the “Carryover From Inadvertent Account” cell in the next day’s spreadsheet. |
| 4 | Print the completed Operations Spreadsheet. - Provide a copy to the Shift Supervisor. - File the Operations Spreadsheet in the Monthly DC Tie folder. |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.