# ERCOT Operations Bulletin 268

ERCOT has revised the Transmission & Security Desk Procedure.  Revision includes updating incorporating CN#301 into procedure as new section 2.3.3.

| ### 2.3.3 Maintenance Level 2 & 3 Outages |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | Maintenance Level 2 and 3 outages are coordinated with the ERCOT Outage Coordination Group. These outages should not be “accepted” by anyone other than that group. However, until they are accepted in the Outage Scheduler, they will continue to show up every time the Outage Scheduler refreshes (that is about every 2 minutes). |
| 1 | DO NOT check the ‘accept’ box for Maintenance Level 2 & 3 Outages in the Outage Scheduler. Instead, place those Maintenance Level 2 & 3 Outages in ‘Study’ status and notify the Outage Coordination group of the Maintenance Level 2 & 3 Outage by sending an email to the ‘1 ERCOT Outage Coordination’ distribution list. |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.