# Terminated 07/05/06

# ERCOT Operations Bulletin 271

CN#358 – Temporary Solution for RPRS Selecting Certain Blackstart Resources

## Effective Date:  June 23, 2006

## Expiration Date:  December 15, 2006

Updated:  June 26, 2006

# Day-Ahead Desk

Introduction:

Blackstart resources are required to be shown in the Resource Plan as offline and available. This makes them eligible for selection by the RPRS market study. Some of these Black Start resources have been frequently selected by the RPRS market study even though the constraint relief provided by these resources has been relatively insignificant to system reliability due to their small size and low shift factors on the constraint.   However their frequent selection and start-up for Replacement will have significant impact on their availability for Black Start service.

A modification to the RPRS study is being written (about 3 months away) that will create an exception list that RPRS will use to selectively ignore certain resources. Operations Support Engineering will update this exception list.

This Temporary Change provides a workaround until the RPRS study is modified.

Procedure:

1. Maintain the most recent exception list provided by Operations Support Engineering behind this page in the Day Ahead Desk section of the Operating Procedures Manual.

NOTE: If other study results conflict with these instructions, take whichever action is determined to be more appropriate, and log the decision if it is different than this procedure requires.

2. Prior to approving an RPRS study (either Day Ahead or Adjustment Period) DESELECT any resource (selected by the RPRS study) that is listed in the exception list provided by Operations Support Engineering.