# ERCOT Operations Bulletin 272

TC # 363 – Fast Start Emergency Schedules to SPP

Effective Date:  June 30, 2006

Expiration Date:  December 30, 2006

Revised Date:  August 9, 2006

Frequency Control Desk

Procedure:

- If SPP’s need for Emergency Power in not immediate, SPP will make arrangements through normal scheduling channels.

- If the need is immediate, SPP will notify ERCOT that emergency assistance in being initiated.

- Before approving the transfer, verify that providing emergency assistance to SPP will not put ERCOT in an insecure state, or worsen an existing insecure state.

- SPP will contact AEP and arrange to maximize DC tie output into SPP.

- It is not necessary to confirm that emergency conditions exist in the RCIS. SPP may need to act before the RCIS is updated.

- Direct the Operating Period Desk to adjust the Offset to minimize the use of Regulation until the schedule is in the MOI (Market Operator Interface).

- A NERC tag will be created after the fact for emergency power.

- Issue VDI (Verbal Dispatch Instruction) to the source QSE (Qualified Scheduling Entity) and include the MW amount along with the start and stop time.

NOTE:

- Every effort should be made to render assistance during emergency conditions.

- The Emergency schedule can start at anytime and is not required to be in the MOI.  A schedule should be put in the ERCOT MOI as soon allowable by the Operating Period Lockout.