- ERCOT Operations Bulletin 278

ERCOT has revised the Shift Supervisor Desk section of the Operating Procedure Manual.

Changes include adding updated step 2 to section 2.2.12 ‘PUC Daily Report.’

| ### PUC Daily Report |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | The PUC reports are to be generated twice per day as outlined below at 1100 and 1500. |
| 1 | Complete the PUCT_Daily Report template at the appropriate times. E-mail the report to the distribution list entitled “PUC-Available Generating Resources.” |
| 2 | Use the following guidelines to determine the potential for EECP when completing the PUCT_Daily Report: - Low - Sufficient capacity to cover peak demand - Medium - An Advisory is in effect for Physical Responsive Reserves - Loss or delay of generation availability during peak demand that may diminish Physical Responsive Reserves - Higher than expected load that may impact reserves - Projected Fuel curtailments - Load forecast indicates a new record peak - High - An Alert is in effect for Physical Responsive Reserves |
| 3 | Revise the last report sent and resend it if: - The probability of an EECP event increases. Give reason(s) for escalation, OR - Conditions change that required new notices to be issued, such as, an OCN or an Alert for generation capacity, OR - Local reliability concerns arise which include actual overloads and/or loss of load, especially the Laredo area, OR - The Gibbons Creek Bryan East/Green Prairie 138 kV double circuit transmission line trips, or if there are unplanned outages of the generation units in the area. |
| NOTE: | Reporting unexpected generation outages is no longer required. However, be able to provide a list of unplanned generation outages for the day if requested by the PUC. |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.