- ERCOT Operations Bulletin 284

ERCOT has revised the Transmission & Security Desk Procedure.  Revision includes updating section 2.4.4 – first Note and step 1.

| ### 2.4.4 VSAT |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| PURPOSE: | To monitor system voltages and to prevent voltage collapse. |
| NOTE: | VSAT should be checked at least every hour, or more frequently as necessary. |
| NOTE: | To contact VSAT support, check the board in the back of TCC for the on-call personnel. |
| 1 | Determine if VSAT has run in the last 80 minutes. Reference Display: Network Online Sequence - If VSAT has not run in the last 80 minutes, contact VSAT support. - IF a South-Houston or North-Houston CSC/CRE has a forced outage, manually rerun the whole sequence of RTNET, RTCA and RTDCP (VSA) immediately and proceed to step 2. Otherwise, continue with following steps. - IF any of the last VSAT results approaches a safety margin, manually rerun the whole sequence of RTNET, RTCA and RTDCP (VSA) within one hour and proceed to step 2. Otherwise, continue with following steps. |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.