- ERCOT Operations Bulletin 289

ERCOT has revised the Operating Desk Procedure.  Revision includes updating section 2.3.1 – step 4 and Appendix 6.

| ### Manage the Real Time Balancing Market |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | Ref Display: Real Time Balancing Market Typically go to “windows” on the tool bar and left click Real Time Balancing Market. |
| 1 | CONFIRM that the real-time balancing energy calculation has completed for the periods ending hh:15, hh:30, hh:45, or hh:00 as needed to assure system is functioning properly. |
| 2 | REVIEW the real-time balancing results. If Ldr = Success, and Net Gen = Net Load 50 MW, go to Step 4. |
| 3 | IF the following conditions apply: - Ldr = Failed - Real Time Balancing Market does not approve, or - If the difference between Net Gen and Net Load is >50 MW Then notify Shift Supervisor and Frequency Control Desk to take action as necessary to maintain system reliability. |
| NOTE: | Verify the generation scheduled in real time balancing matches the generation from the Hour Ahead Study. If the difference is less than +/-100 MWs, determine if new study needs to be rerun. |
| 4 | IF the ‘Price Admin’ box is checked notify Market Participants and the Shift Supervisor. |
| NOTE: | Under the Emergency Notifications tab, post the following message: e.g., “The MCPE for interval <interval> on <day and date> has been deemed by ERCOT to be questionable. Appropriate ERCOT personnel will review the posted value.” |

Appendix 6:  Reviewing Real Time BES Market Information

Review the MCPE for each zone for the interval.  Anytime the ‘Price Admin’ box is checked the MCPE is questionable.

Any questionable or unusual computer system or telemetry failures in a timeframe that may impact calculation of MCPE values should also be considered.

The MCPE is the highest price associated with a Congestion Zone for a Settlement Interval for Balancing Energy deployed during the Settlement Interval.

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.