- ERCOT Operations Bulletin 294

ERCOT has revised the Day Ahead Desk Procedure.  Revision includes updating section 2.11 deleted old step 8; 2.12 step 3, 2.13 step 1 and section 2.15 step 4.

| ## Validate the Resource Plan |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | Steps one thru five should be performed shortly after 0600. |
| 1 | INITIATE the “Studies” display with the appropriate date and intervals to be validated. |
| 2 | VERIFY that the correct input data is displayed on the “System” tab. |
| 3 | Select proper Date and Time range. |
| 4 | SELECT “Validate Resource Plan” in the “Study Mode” area. |
| 5 | SAVE the case with the appropriate name. |
| 6 | By 1615, initiate the Resource Plan validation process. |
| 7 | REVIEW relevant messages and information on the “Control” and/or “Message” tabs. |
| 8 | Click the NOTIFY button. |

| ## Initiate ERCOT Day Ahead Replacement Capacity Market |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 3 | Review Analysis Results - Do NOT deselect any RMR units selected by the RPRS study. Use the study selections to provide an Updated Final RMR Delivery Plan to the appropriate QSE. - Refer to Outage Coordinator Notes for unit requirements due to outages and compare with RPRS results. - If STEP 1 (local congestion)of the RPRS study has selected a steam turbine (ST) from a combined cycle train which requires a combustion turbine (CT) from the same train to be online, it is acceptable to OOMC that CT if it was not already expected to be online or procured. - If Step 2 (zonal congestion and capacity) of the RPRS study has selected a steam turbine from a combined cycle train which requires a combustion turbine from the same train to be online, the market participant must bring on the necessary CT on their own without an OOMC, because resources are usually procured by bids in Step 2. - If Step 1 or 2 selected only combined cycle CT(s), do NOT OOMC its steam units. - Investigate discrepancies, if any. - Confer with the Transmission and Security Operator as necessary on current constraint problems. If the constraints are expected to recur, discuss obtaining generators that would help manage these constraints. Notify the Shift Supervisor. - Deselect additional units for each hour if you know they are not needed. - If deselecting a unit, do NOT replace a de-selection with an OOMC unless you are substituting with another unit for a justifiable reason. |

| ## Initiate ERCOT Day Ahead Replacement Capacity Market |  |  |
| --- | --- | --- |
| ###### Step # | ###### Procedural Steps |  |
| NOTE: | A MCPC value may be considered questionable if it does not follow a reasonable trend when compared to the values immediately preceding or following such a value or a sudden unexplained change occurs in the magnitude of values at any time. |  |
| 1 | By 1800 for a Day Ahead market, - Review the solution of the Replacement Capacity analysis for reasonableness. If capacity was procured in step 2 then spot check Market Clearing Prices - If reasonable, APPROVE, and task is complete. OTHERWISE, CONTINUE - If not reasonable, discuss with Operations Support. If the consensus is that the results are not reasonable, then, either make additional changes and APPROVE OR click UNAPPROVE. - If not completed by 1800, issue an Emergency Notice that this time requirement will not be met. |  |

| ## 2.15 Initiate ERCOT Adjustment Period Replacement Market Study |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 4 | Review Analysis Results - Do NOT deselect any RMR units selected by the RPRS study. Use the study selections to provide an Updated Final RMR Delivery Plan to the appropriate QSE. - Refer to Outage Coordinator Notes for unit requirements due to outages and compare with RPRS results. - If STEP 1 (local congestion)of the RPRS study has selected a steam turbine (ST) from a combined cycle train which requires a combustion turbine (CT) from the same train to be online, it is acceptable to OOMC that CT if it was not already expected to be online or procured. - If Step 2 (zonal congestion and capacity) of the RPRS study has selected a steam turbine from a combined cycle train which requires a combustion turbine from the same train to be online, the market participant must bring on the necessary CT on their own without an OOMC, because resources are usually procured by bids in Step 2. - If Step 1 or 2 selected only combined cycle CT(s), do NOT OOMC its steam units. - Investigate discrepancies, if any. - Confer with the Transmission and Security Operator as necessary on current constraint problems. If the constraints are expected to recur, discuss obtaining generators that would help manage these constraints. Notify the Shift Supervisor. - Review Transmission operator security requirements as received and compare with RPRS results. Request confirmation of the requirements by Operations Engineering if something new or a significant change is seen in the RPRS study results. - De-select additional units for each hour if you know they are not needed. If de- selecting a unit, do NOT replace a de-selection with an OOMC unless you are substituting with another unit for a justifiable reason. |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.