- ERCOT Operations Bulletin 297

ERCOT has revised the Day Ahead Desk Procedure.  Revision includes updating section 2.5 – changed step sequence.

| ## Priority Outages |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | A Priority Outage is one that affects generation. |
|  | Prior to 1600 |
| 1 | - From a review of the Outage Notes for the next Operating Day, make lists of Priority Outages, sorted by affected Transmission Operator. - Save in P:\DAY AHEAD – TDSP PRIORITY OUTAGE NOTIFICATION - Print copies. - Fax the lists to the respective TOs accompanied by a request that within 30 minutes of the Actual Start and Actual End times of these Priority Outages, they enter the Actual Start and Actual End times in the Outage Scheduler - Place in Daily Log File |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.