- ERCOT Operations Bulletin 309

ERCOT has revised the Shift Supervisor Desk.  Changes include updating section 2.2.14.  All steps have been updated and revised.

| ### Provide Advanced Notice of Diminishing Responsive Reserve |  |
| --- | --- |
|  | ###### Procedural Steps |
| PURPOSE: | Guidelines for issuing advance notice of diminishing Responsive Reserves. |
| NOTE: | Adjusted Responsive Reserve (ARR) Reference Display: EMS, Generation Area Status, ADJ_RESPONS |
| 1 | IF Adjusted Responsive Reserve is equal to or less than 3000MW, issue an Advisory. |
| 2 | IF Adjusted Responsive Reserve is equal to or less than 2500MW, issue an Alert. |
| 3 | Start RMR units available in the time frame of the emergency. RMR units should be loaded to full capability. |
| 3a | Issue dispatch instructions to QSEs to suspend any ongoing generating unit or resource performance testing. |
| 3b | Utilize available resources providing Replacement and Non-Spin Reserve Services that can be deployed to increase Responsive reserves. |

A copy of the procedure is available at http://www.ercot.com/mktrules/guides/procedures/index.html.