ERCOT Operations Bulletin 311

ERCOT has revised the Frequency Control Desk.  Changes include updating step 1 and removing step 3.

| ### Managing and Documenting Daily Inadvertent and Payback South DC Tie |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| OVERVIEW: | Inadvertent Energy flows across the DC Tie need to be monitored frequently enough during the day to ensure that large Inadvertent Energy flows don’t accumulate. |
| NOTE: | Payback will only be scheduled when there is room on the tie. Power schedules of market participants will supersede any inadvertent payback schedule. |
| NOTE: | Normally no MWs will be scheduled on the DC Tie. All the power that flows will be treated as inadvertent. |
| 1 | After Midnight: - Determine if the daily “Total Actual Flow” values in WebTrans agree with the AEP Corpus Christi TDSP total. If, not, then correct Webtrans as necessary. |
| 2 | Verify planned flows for the next day with the AEP Corpus Christi TDSP Operator. |

A copy of the procedure is available at

http://www.ercot.com/mktrules/guides/procedures/index.html.