ERCOT Operations Bulletin 324

ERCOT has revised the Operating Period Desk sections of the Operating Procedure Manual.

This change updates:

Section 3.4 – Appendix 4:  Added detail on How to Deploy/Recall NSRS

A copy of the procedure is available at

http://www.ercot.com/mktrules/guides/procedures/index.html

3.4	Appendix 4:  How to Deploy/Recall NSRS

DEPLOY NSRS

Reference Displays:  Non-Spin Deployment display in MOI.

NOTE:  IF NSRS is scheduled and available, deploy for a minimum of one hour and continue as needed.

If NSRS is necessary and is scheduled and available, communicate with the Shift Supervisor to ensure an active constraint won’t be negatively impacted, then deploy NSRS as scheduled and needed to resolve UBES deficiency.

To open the Non-Spin window in the ERCOT Market Operator Interface (MOI), SELECT WINDOW on control bar at the top of the MOI and select the “Non-Spin Deployment” from the scroll down menu.

SELECT the appropriate “Date” and “Zone”:

    - RIGHT CLICK in the DATE box.  POP-UP calendar will appear, LEFT CLICK the desired date and SELECT “OK” to accept date or CANCEL to close POP-UP box

OR

    - Left click in the “Date” field and use the up and down arrow keys to select desired date.

    - Left Click the “Zone” field drop down arrow and select the desired zone to view the scheduled MW amount and hours scheduled.  (If deployments are needed in more than one zone, repeat this step for each zone to obtain the scheduled MW amounts and hours scheduled)

CLICK on the GREEN PLUS (+) on the tool bar at top of page, which will add a blank row.

      - Right Click on “Zone” field, Select “List Values”, select the appropriate zone.

      - Right Click on “Start Date” field, Select “List Values”, select the current date from calendar.

      - Left Click drop down arrow for “Start Time” and select desired start time.

      - Right Click on “End Date” field, Select “List Values”, select the current date from calendar.

      - Left Click drop down arrow for “End Time” and select desired end time.

      - Enter in the desired MW value (value has to be equal to or less than the amount scheduled for that zone).

      - Enter Deploy NSRS in the “Comments” field.

      - Left Click in the “Posted Time” field, then arrow down and back up to display the current date and time.

      - Press F10 to save and send out deployments.

      - Verify deployments sent by viewing the “Zonal Deploy” column.

      - Repeat process for each zone that NSRS is to be deployed in.

RECALL NSRS

As system conditions permit or as outlined in Section 2.1.2 of the Operating Period desk procedures, allow NSRS deployments to expire or recall NSRS prior to scheduled end time.

NOTE:  Allowing NSRS deployments to expire at the scheduled end time requires no Operator action.

Manual recall of NSRS deployments prior to scheduled end time.

CLICK on the GREEN PLUS (+) on the tool bar at top of page, which will add a blank row.

      - Right Click on “Zone” field, Select “List Values”, select the appropriate zone.

      - Right Click on “Start Date” field, Select “List Values”, select the current date from calendar.

      - Left Click drop down arrow for “Start Time” and select desired start time.

      - Right Click on “End Date” field, Select “List Values”, select the current date from calendar.

      - Left Click drop down arrow for “End Time” and select desired end time.

      - Enter a zero in the “MW” field if recalling total amount of deployed NSRS (Enter the desired MW amount for partial recall of deployed NSRS).

      - Enter Recall NSRS in the “Comments” field if deployments are going to zero (Enter Deploy NSRS in the “Comments” field for partial recall of deployed NSRS).

      - Left Click in the “Posted Time” field, then arrow down and back up to display the current date and time.

      - Press F10 to save and send out deployments.

      - Verify deployments recalled by viewing the “Zonal Deploy” column.

      - Repeat process for each zone that NSRS is to be recalled in.