## Power Operations Bulletin # 383

ERCOT has revised the Day-Ahead Desk Procedure Manual.

The specific changes are shown below.

A copy of the procedure can be found at:

http://www.ercot.com/mktrules/guides/procedures/index.html

| ## 2.3 Enter A/S Requirements & Review the System Conditions Forecast |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTES: | - Step 1 must be completed prior to 0600. - The Shift Supervisor must verify the posted ancillary service requirements for the 1st day of the month. |
| 1 | Verify and PUBLISH the hourly ERCOT system total Ancillary Service requirements for each of the following, for each hour of the operating day: (Refer to Appendix 3 if desired) - DRS - URS - RRS - NSRS (See Appendix 4) |
| NOTE: | Step 2 may be completed after 0600. |
| 2 | VERIFY by spot-checking the following information posted to the ERCOT Market User Interface (the Texas Market Link) or the ERCOT Public Web for the next operating day (or two days if two-day Ahead market is in effect) is posted. - Planned Transmission line and equipment outages for the operating day. - Texas Market Link; Market Activities Tab; Outage Scheduler; Outage Scheduler; OR https://mostml.ercot.com/outages_prp/default.cfm OR - www.ercot.com; Grid Information Tab; Approved Transmission Outages link OR http://mospublic.ercot.com/ercot/jsp/planned_trans_out.jsp - MOI: Daily Operations Plan; Outage Schedule tab - Weather assumptions used to provide the system conditions forecast. - www.ercot.com; Grid Information Tab; Weather; OR http://mospublic.ercot.com/ercot/jsp/tem_peaks.jsp - MOI: No information - The forecast of hourly loads by Congestion Management Zones and hourly ERCOT System loads - Market Information Tab; Load Data link; Midterm Load Forecast OR http://mospublic.ercot.com/ercot/jsp/midterm_load_forecast.jsp OR - Texas Market Link; Market Activities Tab; Market Information; View Midterm Load Forecast - The hourly ERCOT System total Ancillary Service requirements for Regulation Down – DRS, Regulation Up – URS, Responsive Reserve – RRS, and Non-spinning Reserve – NSRS. - Texas Market Link; Market Activities Tab; Market Information; Ancillary Service Requests OR - Market Information Tab; Energy and Ancillary Services link; Ancillary Services Bid Requests link; OR http://mospublic.ercot.com/ercot/jsp/ancillary_service_requests.jsp |