Power Operations Bulletin #412

ERCOT has revised the Day-Ahead Desk Procedure Manual.

The specific changes are shown below.

A copy of the procedure can be found at:

http://www.ercot.com/mktrules/guides/procedures/index.html

## 2.4	Provide Initial Delivery Plan to QSEs Representing RMR and/or Synchronous Condenser Units

Procedure Purpose: This task is performed to notify QSEs of RMR resources that may be required for the day ahead by using the current day’s actual selections as the basis for preliminary selections for the Day Ahead.

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| NOTE: | The Shift Supervisor and Day Ahead desk should receive, from the RMR resource’s QSE each day by 0600 via email, a spreadsheet showing the availability of each RMR unit on a tab labeled “Availability.” |
| 1 | By 0900: IDENTIFY the RMR resources that were selected for possible operation in the current operating day. - Based on current system conditions and forecasts and the “Availability” provided in the spreadsheet, - SELECT the tab labeled “Initial Delivery Plan” and FILL IN the best estimate of the next day’s delivery requirements for secure operations. - The MW values entered should be within the unit’s Low Sustained Limit (LSL) and the High Sustained Limit (HSL). - Save the file to the System Operations server (P:/Day Ahead-RMR Delivery Plans) - EMAIL the file to the e-mail group associated with the resource: QSEs whose resources are included in the Initial Delivery Plan, ERCOT Shift Supervisor ERCOT Day-Ahead Desk (Print and fax if e-mail is not working.) |
| Example: | E-mail group “RMR *** 0900” sends mail to Market Participant (***) QSE, Day-Ahead Desk and Shift Supervisor. |

## 2.7	Procure Ancillary Services (DRS, URS, RRS, NSRS) in Day Ahead

Procedure Purpose:

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| 1 | Between 1315 and 1330, CALL-UP the “Ancillary Service Markets” display. |
| 2 | VERIFY that the correct date for the operating day is selected. |
| 3 | SELECT “Execute.” |
| 4 | When execution is complete, REFRESH display (F8). If procurement of Ancillary Services is not complete by 1330, proceed to Step 6. |
| 5 | Review and Approve “Refer to 2.8 Review the Ancillary Services Market Clearing Data for the ERCOT System.” |
| 6 | If the procurement of Ancillary Services is not completed by 1330, issue an Market Emergency Notice to QSEs, using the hotline: - When QSEs have answered the Hotline, print Hotline Log. Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing a Market Emergency Notice. Procurement of Ancillary Services has not met the time requirement. ERCOT will notify you when more information is available. [QSE] please repeat this back to me. That is correct, thank you.” - Post the following message on the MIS and ENS: - A Market Emergency Notice issued due to the procurement of Ancillary Services has not met the time requirement”. |
| 7 | Once the procurement of Ancillary Service is complete, terminate the Market Emergency Notice to QSEs, using the hotline: - When QSEs have answered the Hotline, print Hotline Log. Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. The procurement of Ancillary Services has completed. ERCOT is terminating the Market Emergency Notice. [QSE] please repeat this back to me. That is correct, thank you.” - Update messages on the MIS and ENS: - MIS – Make Market Emergency Notice invalid. - ENS – Enter end time for Market Emergency Notice. |

## 2.8	Review the Ancillary Services Market Clearing

Procedure Purpose:

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| NOTE: | A capacity shortage is significant if it cannot be accounted for by system rounding errors or known problems in ERCOT or QSE systems. |
| 1 | Check for any significant capacity shortage for any hour for DRS, URS, RRS, and NSRS. MOI: “Ancillary Service Markets” – “Ancillary Service” tab: If a capacity shortage exists for any ancillary service for any hour, THEN CONTINUE; OTHERWISE, GO TO Step 3. |
| 2 | If any significant shortages exist for any Ancillary Service, notify the Shift Supervisor and Market Operations personnel. Issue a Watch to QSEs, using the Hotline: - When QSEs have answered the Hotline, print Hotline Log. Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing a Watch. [Ancillary Service(s)] bids were insufficient for [MM/DD/YYYY], hour(s) ending [HE]. ERCOT will [open a second Ancillary Service Market] or [procure additional capacity]. [QSE] please repeat this back to me. That is correct, thank you.” Post message on the MIS and ENS: - MIS – “A Watch has been issued due to insufficient [Ancillary Service(s)] bid for [MM/DD/YYYY], hours ending [HH].” - ENS – “Watch for Insufficient Ancillary Service bids, see MIS”. IF adequate Emergency Short Supply Services are procured, THEN TERMINATE the Watch. OTHERWISE, CONTACT the Shift Supervisor for resolution. |
| 3 | Once the shortage(s) of Ancillary Service has ended, terminate the Watch to QSEs, using the hotline: - When QSEs have answered the Hotline, print Hotline Log. Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is terminating the Watch for insufficient Ancillary Services. [QSE] please repeat this back to me. That is correct, thank you.” - Update messages on the MIS and ENS: - MIS – Make Watch Notice invalid. - ENS – Enter end time for Watch Notice. |
| NOTE: | A MCPC value may be considered questionable if it does not follow a reasonable trend when compared to the values immediately preceding or following such a value or a sudden unexplained change occurs in the magnitude of values at any time. |
| 4 | REVIEW the Market Clearing Prices for Capacity for each hour of the operating day for DRS, URS, RRS, and NSRS. IF any MCPC values appear questionable, GO TO next Step. OTHERWISE, DONE. MOI: Ancillary Services Market display; Ancillary Services tab; OR http://mospublic.ercot.com/ercot/jsp/ancillary_services_mcp.jsp; and select operating day. |
| 5 | DISCUSS with the Shift Supervisor and Market Operations Support personnel. IF it is determined that one or more MCPC values are questionable: Post message on the MIS and ENS: - MIS – “The MCPC for [ancillary service] for hour [hour] on [day and date] has been deemed questionable by ERCOT. Appropriate ERCOT personnel will review the posted value.” - ENS – “MCPC questionable for [hour] on [DD/MM/YY], see MIS”. |

## 2.9	Verify Ancillary Service Schedules Updated

Procedure Purpose: To verify the A/S schedules are updated and identify deficits

| Protocol Ref. | 6.5.5 |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| NOTE: | QSE’s planning to offer Balancing Energy Service Capable NSRS (BESCNSRS) into the Balancing Energy Market, must adjust their NSRS Resources and Obligations by removing the amount that is supplied by BESCNSRS and must provide Balancing Energy Service Up bids after the Ancillary Service market clearing. There are no changes needed for the portion of NSRS schedule supplied for 30 Minute NSRS. |
| 1 | Around 1445, CHECK that QSE schedules are updated for Up Regulation, Down Regulation and Responsive Reserve. If not, REMIND the QSEs that schedules are required to be updated by 1500. |
| 2 | Around 1515, VERIFY that no Ancillary Service deficits exist for Up Regulation, Down Regulation and Responsive Reserve by ensuring that scheduled Ancillary Services match obligations. |
| 3 | For NSRS, QSE’s must adjust their scheduled Resource and Obligation by the amount of their BESCNSRS commitments. These adjustments will indicate a deficiency in the MOI. Differences in the NSRS will need to be verified in the Excel MAI. To ensure that sufficient NSRS is scheduled and no deficits exist, verification in the Excel MAI is required. If there are no deficits, GO TO Section 2.11. If deficits exist, CONTINUE |
| 4 | It will not always be possible to identify a specific QSE at this point. |
| 5 | To attempt to IDENTIFY the QSE that has not scheduled its Ancillary Services Obligations for Up Regulation, Down Regulation and Responsive Reserve: MOI: Ancillary Services Market display; QSE Schedule Pool tab; right click on desired hour, Select AS QSE Pool Summary on menu. (Hint: Look for QSEs with Ancillary Service awards quantities equal to the amount of capacity shortage.) For NSRS, attempt to IDENTIFY the QSE that has not scheduled its obligations in the Excel MAI by utilizing the Excel MAI and QSE Schedule Pool tab. If QSE is not identified, discuss with Shift Supervisor and/or Market Operations. - COMMUNICATE with the QSE about the problem and request that the schedule be corrected. - IF the QSE reports it is unable to schedule its obligations because it is short of resources, then REQUEST that they purchase from the market to meet their obligations. - IF the problem is corrected by 1600, then PROCEED to Section 2.11 otherwise continue. - IF the cause of the Ancillary Service capacity shortage is determined to be in the ERCOT systems, THEN LOG that the resources are available, and go on to Section 2.11. - DISCUSS with the Shift Supervisor (the Shift Supervisor should consult with the Manager, System Operations and Market Operations Support) and decide if opening a new Ancillary Services market for the deficit amount is likely to resolve the problem. If not, go to Section 2.11. OTHERWISE CONTINUE. - IF opening a new market is determined to be the solution, then open a new market for the total requirement. - At least 10 minutes prior to a clock hour, but no less than two hours prior to the Operating Hour that the Ancillary Services capacity is required, - On the “Ancillary Service Markets” page; RIGHT CLICK anywhere in the first row of fields at the top of the screen. - SELECT “Open a new Ancillary Services Market.” - SET the bid open time equal to the top of the clock hour and the bid close time equal to 30 minutes past the top of the clock hour. - CONFIRM the amount of Ancillary Services required remains unchanged from the previous market. - At five minutes prior to the top of the clock hour, PUBLISH Ancillary Services Obligations. - At the top of the clock hour, POST a Market Insufficiency Watch that says: - MIS - “ERCOT opened at <top of the clock hour> an Ancillary Services market for <applicable services> for the sole reason of resolving a default condition during the hours of <applicable hours>. Bids will be received between <top of the clock hour> to <30 minutes past the top of the clock hour>. Any bids eligible for removal may be removed between the<top of the clock hour> to <30 minutes past the top of the hour>. This does not affect Ancillary Services self-arranged, awarded or scheduled from previous markets.” - ENS – “Watch for Market Insufficiency, See MIS.” - Issue a Hotline call to QSEs with a message similar to: Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing a Watch. Additional Ancillary Services are needed. ERCOT will open at [top of the clock hour] an Ancillary Services market for [applicable services] for the sole reason of resolving a default condition during the hours of [applicable hours]. Bids will be received between [top of the clock hour] to [30 minutes past the top of the clock hour]. Any bids eligible for removal may be removed between the [top of the clock hour] to [30 minutes past the top of the hour]. This does not affect Ancillary Services self-arranged, awarded or schedule from previous markets. [QSE] please repeat this back to me. That is correct, thank you.” - At 45 minutes past the top of the hour, PROCURE Ancillary Services. |
| 6 | About 30 minutes after market clearing, CHECK that QSE schedules are updated. If not, REQUEST that schedules be updated by one hour after market clearing. |
| NOTE: | A Resource Plan Validation Study may be run if desired following step 7 below. |
| 7 | About one hour after market clearing, VERIFY that no Ancillary Service deficits exist by ensuring that scheduled Ancillary Services match obligations. If deficits exist, return to Step 5. Otherwise, GO TO Section 2.11. |

## 2.10	Procure Additional Ancillary Services (DRS, URS, RRS, NSRS) during the Adjustment Period

Procedure Purpose:  Additional A/S needed in the Adjustment Period

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| 1 | If following the close of the Day-Ahead Market, conditions develop such that additional Ancillary Services may be needed, DISCUSS with the Shift Supervisor. If it is determined that an additional market is NOT needed, then LOG the decision and continue with normal duties. OTHERWISE, CONTINUE. |
| NOTE: | The Shift Supervisor should consult with the appropriate Manager and Market Operations Support to decide if opening an Adjustment Period market for additional Ancillary Services is appropriate. Remember that QSE’s scheduling BESCNSRS into the BES market must adjust their NSRS Resource and Obligation schedules by the amount that is supplied by BESCNSRS and must provide BES Up bids after the Ancillary Service market clearing. |
| NOTE: | Time X in this procedure is the start of the clock hour after it is determined that an additional Ancillary Services market is appropriate. |
| 2 | By Time X (no less than three hours prior to the start of the Operating Hour that the Ancillary Service capacity is required), using the hotline, notify QSEs: - When QSEs have answered the Hotline, print Hotline Log. Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing a Watch. Additional Ancillary Services are needed. At [X+60] ERCOT will open an Adjustment Period Ancillary Services market for [applicable services] during the hours of [applicable hours]. Bids eligible for removal may be removed between [X] and [X+60]. [QSE] please repeat this back to me. That is correct, thank you.” - Post message to MIS: ERCOT issued a Watch. Additional Ancillary Services are needed. At [X+60] ERCOT will open an Adjustment Period Ancillary Services market for [applicable services] during the hours of [applicable hours]. Bids eligible for removal may be removed between [X] and [X+60]. - Post message to ENS: “Watch – Insufficient Ancillary Services, see MIS” |
| 3 | At <X+55> CREATE a new Ancillary Services market. - SET the start and end times for the market to match the period for which additional Ancillary Services are needed. - SET the bid open time equal to <X+60>. SET the bid close time equal to <X+90>. - INCREASE the amount of Ancillary Services to equal the quantity from the previous market plus the additional amount required for applicable services during the applicable hours except for NSRS. For NSRS you must increase the requirement the amount of the requirement that you are short plus any additional amount required. Example: Previous market requirement + additional quantity needed = quantity to enter. NSRS example: Previous market requirement – scheduled + difference + additional quantity needed = quantity to enter. |
| 4 | At <X+60> PUBLISH Ancillary Service Obligations. |
| 5 | At <X + 90>, PROCURE and APPROVE this market. |
| 6 | CHECK that QSE schedules are updated. If not, then REQUEST that the schedules be updated by one hour after market clearing. |
| 7 | - About one hour after market clearing, VERIFY that no Ancillary Service deficits exist. - A Resource Plan Validation Study may be run if desired. - If deficits exist, notify Shift Supervisor and/or Market Operations Support. |

## 2.12	Initiate ERCOT Day Ahead Replacement Capacity Market

Procedure Purpose:

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| 1 | - Before 0600, on the Day Ahead Market Management Display, CONFIRM a Replacement Market was created for tomorrow for hours 0100 to 2400. - If NOT, then NOTIFY EMMS Production or the Helpdesk and manually create the RPRS market for tomorrow, OTHERWISE CONTINUE. |
| NOTE: | Ensure OOMC for Voltage Support (if applicable) are complete prior to Step 2. This includes units for voltage support from the Outage Notes. |
| NOTE: | QSEs that plan to provide Non-Spin from off-line GTs that can start in 30 minutes or less, should have their Non-Spin flag checked in order to identify the off-line units being used for Non-Spin. RPRS study will ignore “On-line and Available” units and units with lead time longer than 30 minutes that have their Non-Spin flags checked. RPRS study will consider the off-line GTs as on-line if their Non-Spin flag is checked. The Non-Spin obligation will be added to the Replacement System Capacity Obligation as well. |
| NOTE: | If it is determined that deficiencies exist with the input data into RPRS (i.e. Incorrect or Invalid Resource Plan, Invalid Load Forecast, etc), and these deficiencies may potentially affect the RPRS solution, additional actions may need to be taken. If RPRS completes it may result in excessive procurement of Step 2 capacity. |
| 2 | Make sure that the “Execute W/L” checkbox is checked. If it is not, then click on it to check the box. |
| 3 | After the Resource Plan is validated, RUN Steps 1 and 2 of Replacement Analysis by clicking the “EXECUTE” button. - If a QSE notifies ERCOT of a Resource Plan problem after the Study has begun, THEN: - Consult with the Shift Supervisor to determine whether or not to stop RPRS. - If the decision is to let RPRS complete on its own, THEN do so and go to Step 4 of this procedure. Otherwise, CONTINUE. - If the decision is to stop RPRS, THEN - STOP RPRS, and wait for the Resource Plan to be updated by the QSE. - Re-run RPRS once the Resource Plan is updated. |
| NOTE: | Resource de-selection and re-selection is done on the Procurement Summary Page. |
| 4 | Review Analysis Results - Do NOT deselect any RMR units selected by the RPRS study unless there is a valid reason. Use the study selections to provide an Updated Final RMR Delivery Plan to the appropriate QSE. - Refer to Outage Coordinator Notes for unit requirements due to outages and compare with RPRS results. - If STEP 1 (local congestion) of the RPRS study has selected a steam turbine (ST) from a combined cycle train which requires a combustion turbine (CT) from the same train to be online, it is acceptable to OOMC that CT if it was not already expected to be online or procured. - If Step 2 (zonal congestion and capacity) of the RPRS study has selected a steam turbine from a combined cycle train which requires a combustion turbine from the same train to be online, the market participant must bring on the necessary CT on their own without an OOMC, because resources are usually procured by bids in Step 2. - In Step 1 or Step 2, if the RPRS study has selected a combustion turbine (CT) from a combined cycle train which requires a steam turbine (ST) from the same train to be online, it is acceptable to OOMC a ST if it was not already expected to be online or procured. - Investigate discrepancies, if any. - Deselect additional units for each hour if you know they are not needed. - If deselecting a unit, do NOT replace a de-selection with an OOMC unless you are substituting with another unit for a justifiable reason. |
| 5 | Execute RPRS Step 3 (Pricing). |

## 2.13	Procure Replacement Reserve Services and Manage Capacity Shortfalls

Procedure Purpose:

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| NOTE: | If it is determined that weather, outage or other conditions will require capacity from resources with lead times too long to be considered by RPRS, then additional capacity should be OOMCd via Verbal Dispatch Instruction. The choice of resources to OOMC should be made considering the causative conditions, available resources, lead times, congestion management and other operational concerns. |
| 1 | By 1800 for a Day Ahead market, - Review the solution of the Replacement Capacity analysis for reasonableness. If capacity was procured in step 2 then spot check Market Clearing Prices - If reasonable, APPROVE, and task is complete. OTHERWISE, CONTINUE - If not reasonable: - Verify whether or not the Resource Plan has been corrected by the QSE. - If the Resource Plan has been corrected, THEN re-run RPRS. - If the Resource Plan has not been corrected, THEN contact the QSE and determine how long it will take for the Resource Plan to be updated. Once the Resource Plan is updated, THEN re-run RPRS. - If the result is reasonable, THEN initiate Step 3 (Pricing) and APPROVE the study and go on to STEP 2. Otherwise, CONTINUE - Discuss with Operations Support. If the consensus is that the results are not reasonable, then, either make additional changes and APPROVE OR click UNAPPROVE. - If it is determined in any of the steps above that a valid RPRS result will not be achieved when Step 1 and 2 of RPRS completes, UNAPPROVE the study. - If the RPRS study completes in a “Failed” state, click on the “Execute” button, and re-run RPRS, if time permits. If time does not permit, click on the “UnApprove,” button. Immediately contact EMMS Production Support On-Call staff, and have them determine, and fix, the problem that caused the RPRS study to fail. - If not completed by 1800, issue a Market Emergency Notice that this time requirement will not be met. Using the hotline, notify all QSEs of the Emergency Notice that the RPRS time requirement will not be met Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing a Market Emergency Notice. Replacement Reserve Service (RPRS) has not met the time requirement for Operating Day [XX/XX/XXXX]. ERCOT will notify you when more information is available. [QSE] please repeat this back to me. That is correct, thank you.” - Post message on the MIS and ENS: - Market Emergency Notice issued due to RPRS has not met the time requirement. |
| 2 | Following any procurement or non-approval of replacement capacity, determine the OOM/RMR units to be manually dispatched if the market cannot resolve a capacity shortage. - IDENTIFY any expected capacity shortages for any hour(s) of the operating day. - IF a capacity shortage for a given hour is insignificant (i.e., less than 100 MW), RECOMMEND to the Shift Supervisor that it be observed during the adjustment period for possible correction. - IF the shortage is significant (i.e., greater than 100 MW), confer with the Transmission & Security desk operator about constraints and consider OOM for enough capacity in any zone to make up the shortage for the hours needed. Minimize starts in the OOMC process. - IF a QSE refuses to commit capacity selected in step 2 of the RPRS study, OOMC equivalent MW capacity elsewhere. Log the event & notify the shift supervisor. |

## 2.14	Initiate ERCOT Adjustment Period Replacement Capacity Advisory Study

Procedure Purpose:

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| CAUTION | When an ERCOT Adjustment Period Replacement Capacity STUDY is run, Do NOT select the PUBLISH button on the Study page before you execute the study. Selecting the PUBLISH button on the Study page will OPEN an RPRS Market. |
| 1 | After the close of the Day Ahead Market, as time permits or conditions develop such that additional Replacement Reserve Services may be needed; INITIATE the “Studies” display the appropriate date and intervals to be studied. - Ensure “Execute SFT” is checked. - Ensure “Execute W/L” is checked. - SELECT “Replacement” in the “Study Mode” area. - SELECT the appropriate hours through hour ending 2400 of the day of the study SAVE the study with the appropriate name. |
| 2 | RUN a replacement analysis by clicking the “EXECUTE” button. |
| 3 | Review the results to determine if an ERCOT Adjustment Period Replacement MARKET is needed. DISCUSS with the Shift supervisor when necessary. |
| 4 | LOG the time the Study was initiated and any problems including any actions taken. |

## 2.15	Initiate ERCOT Adjustment Period Replacement Market

Procedure Purpose:

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| NOTE: | If either the Excel MAI or the results of section 2.13 indicates that an additional Replacement MARKET is needed, DISCUSS with the Shift Supervisor. Time permitting, NOTIFY the Shift Supervisor if the projected shortage is based on a reasonable load forecast and a potential capacity shortage still exists on Excel MAI after an Adjustment Period Replacement Capacity STUDY completes. If it is determined that a second Market is needed, CONTINUE. |
| NOTE: | In the following steps, “X” designates the time at which the need for additional RPRS is identified (adjusted to the top of the next clock hour). Protocol requires no less than two hours, but that has proven to be impractical. |
| 1 | At least 10 minutes prior to time X, but no less than three hours prior to the start of the first Operating Hour that the additional Replacement Reserve Services are required, OPEN a RPRS market by selecting the PUBLISH button on the Study page for the appropriate hours through hour ending 2400 of the day of the study. |
| 2 | AT time X, ISSUE a WATCH using the hotline, notify QSEs: - When QSEs have answered the Hotline, print Hotline Log. Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing a Watch. Conditions have developed that additional Replacement Reserve Services are needed. At [top of hour], ERCOT will open an Adjustment Period Replacement market for the remainder of the day. QSEs have 30 minutes to update their schedules. ERCOT reminds Wind QSEs to update their resource plans with the latest AWS Truewind forecast. [QSE] please repeat this back to me. That is correct, thank you.” Post message on the MIS and ENS: - MIS – “ERCOT has issued a Watch. Additional Replacement Reserve Services are needed. At <top of clock hour> ERCOT opened an Adjustment Period Replacement market for the remainder of the day. QSE’s have 30 minutes to update their schedules before ERCOT purchases additional Replacement Reserves. Wind QSEs are reminded to update their resource plans with your latest AWS Truewind forecast.” - ENS – “Watch for Adjustment Period Replacement Market, see MIS”. |
| 3 | At time X+ 45 minutes, RUN a replacement analysis by clicking the “EXECUTE” button. |
| 4 | Review Analysis Results - Do NOT deselect any RMR units selected by the RPRS study unless there is a valid reason. Use the study selections to provide an Updated Final RMR Delivery Plan to the appropriate QSE. - Refer to Outage Coordinator Notes for unit requirements due to outages and compare with RRS results - If STEP 1 (local congestion) of the RPRS study has selected a steam turbine (ST) from a combined cycle train which requires a combustion turbine (CT) from the same train to be online, it is acceptable to OOMC that CT if it was not already expected to be online or procured. - If Step 2 (zonal congestion and capacity) of the RPRS study has selected a steam turbine from a combined cycle train which requires a combustion turbine from the same train to be online, the market participant must bring on the necessary CT on their own without an OOMC, because resources are usually procured by bids in Step 2. - In Step 1 or Step 2, if the RPRS study has selected a combustion turbine (CT) from a combined cycle train which requires a steam turbine (ST) from the same train to be online, it is acceptable to OOMC a ST if it was not already expected to be online or procured. - Investigate discrepancies, if any. - Review Transmission operator security requirements as received and compare with RPRS results. Request confirmation of the requirements by Operations Engineering if something new or a significant change is seen in the RPRS study results. - De-select additional units for each hour if you know they are not needed. If de- selecting a unit, do NOT replace a de-selection with an OOMC unless you are substituting with another unit for a justifiable reason. |
| 5 | Execute RPRS Step 3 (Pricing). |
| NOTE: | A MCPC value may be considered questionable if it does not follow a reasonable trend when compared to the values immediately preceding or following such a value or a sudden unexplained change occurs in the magnitude of values at any time. Inform Market Operations Support of questionable MCPC values. |
| 6 | By time X + 60, review the solution of the Replacement Capacity analysis for reasonableness. - If reasonable, APPROVE, and task is complete. OTHERWISE, CONTINUE If not reasonable, discuss with Operations Support. If the consensus is that the results are not reasonable, then, click UNAPPROVE. |
| 7 | Following any procurement or non-approval of replacement capacity, determine the OOM/RMR units to be manually dispatched if the market cannot resolve a capacity shortage. - IDENTIFY any expected capacity shortages for any hour(s) of the operating day. - IF a capacity shortage for a given hour is insignificant (i.e., less than 100 MW), RECOMMEND to the Shift Supervisor that it be observed during the adjustment period for possible correction. - IF the shortage is significant (i.e., greater than 100 MW), and there are no constraints, OOM enough capacity in any zone to make up the shortage for the hours needed. Minimize starts in the OOMC process. |

## 2.16	Prepare the Day-Ahead Summary Report (Prior to end of shift)

Procedure Purpose:

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| 1 | Complete Day Ahead Summary Report. Refer to Section 6 of the Desktop Reference Guide: ERCOT Day-Ahead Report (Typical) for an example. Include a print-out of the RPRS Spreadsheet if RPRS was procured. |
| 2 | PASS the Day Ahead Summary Report to the Shift Supervisor and brief the Shift Supervisor on what replacement was procured for next day, OOMC and RMR. |

## 2.17	Provide an Updated Delivery Plan to QSEs Representing RMR

Procedure Purpose: This task is performed to notify QSEs of RMR resources that are required for the day ahead.

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| 1 | By 1630, DETERMINE, by referring to the System Area Requirements, which RMR resources and synchronous condenser units may be required for each hour of the operating day. - On the Availability spreadsheet, SELECT the tab labeled “Delivery Plan” and FILL IN the next day’s delivery requirements for secure operations. - The MW values entered should be within the unit’s Low Sustained Limit (LSL) and the High Sustained Limit (HSL). - SAVE the file to the “System Operations” server (P:/drive) in the “RMR Delivery Plan” folder. - EMAIL the file to the e-mail group associated with the resource: QSEs whose resources are included in the Final Delivery Plan, ERCOT Internal Settlements ERCOT Shift Supervisor ERCOT Day-Ahead Desk (Print and fax if e-mail is not working.) |
| Example: | E-mail group “RMR *** 1630” sends mail to Market Participant (***), Internal Settlements, Day-Ahead Desk, Shift Supervisors and various internal ERCOT personnel. |
| 2 | SEND the revised spreadsheet to the QSE and ERCOT Settlements via email distribution list above. (Print and fax if e-mail is not working.) |

## 2.18	Extend, Adjust, or Suspend the Day-Ahead Scheduling Process

Procedure Purpose:

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| 1 | IF extending the Day-Ahead scheduling process to a two Day-Ahead process has been authorized by the Shift Supervisor, Manager of System Operations and Market Operations, ADJUST the timing accordingly and: Issue a Market Emergency Notice and notify the QSEs using the Hotline - When QSEs have answered the Hotline, print Hotline Log. Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing a market emergency notice extending the Day-Ahead scheduling process to a two day ahead scheduling process due to [reason]. [QSE] please repeat this back to me. That is correct, thank you.” Post message on the MIS and ENS: - MIS – “ERCOT is issuing a market emergency notice extending the Day-Ahead scheduling process to a two day ahead scheduling process due to [reason].” - ENS – “Market Emergency Notice issued due extending to a two Day-Ahead process, see MIS”. |
| 2 | IF a change to the Day-Ahead scheduling process timing has been authorized by the Shift Supervisor, Manager of System Operations and Market Operations, ADJUST the timing accordingly and: Issue a Market Emergency Notice and notify the QSEs using the Hotline - When QSEs have answered the Hotline, print Hotline Log. Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing a market emergency notice. The Day-Ahead scheduling process is being adjusted to [time/hour] due to [reason]. [QSE] please repeat this back to me. That is correct, thank you.” Post message on the MIS and ENS: - MIS – “ERCOT is issuing a market emergency notice and is adjusting the Day-Ahead scheduling process to [time/hour] due to [reason].” - ENS – “Market Emergency Notice issued due adjusting the Day-Ahead scheduling process, see MIS”. |
| 3 | IF the Day-Ahead Market software fails without expectation of timely repair or some emergency event (Black Start, etc.) makes market operations impossible THEN, notify Manager, System Operations and Market Operations: If Day-Ahead Market is Suspended, issue an emergency notice and notify QSEs using the hotline call. This call will include - When QSEs have answered the Hotline, print Hotline Log. Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing an emergency notice suspending the Day-Ahead Market due to [reason]. Interim instructions are to [instruction]. The estimated duration of this event is [duration]. [QSE] please repeat this back to me. That is correct, thank you.” Post message on the MIS and ENS: - MIS – “The ERCOT market software has failed and is not expected to function in time to clear the market for <MM/DD/YY>. ERCOT therefore declares a market emergency and directs all QSEs to maintain Ancillary Services obligations as detailed for <PREVIOUS DAY> in the coming market period.” - ENS – “Emergency Notice issued due to Market failed, see MIS”. |
| 4 | If the clearing of the Day Ahead Ancillary Services market fails, notify Manager, System Operations and Market Operations. DISCUSS with EMMS Production Support whether to APPROVE or UNAPPROVE the market results. THEN: IF UNAPPROVED, notify QSEs using the hotline: Typical script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. ERCOT is issuing an emergency notice extending the Day-Ahead Market due to ERCOT’s Day Ahead Ancillary Services market failing to clear. The market will be extended until further notice. Do not resubmit bids or make changes until further notice. [QSE] please repeat this back to me. That is correct, thank you.” Post message on the MIS and ENS: - MIS – “The Day Ahead Ancillary Services market has failed to clear. ERCOT therefore declares a market emergency. The market will be extended until further notice. Do not resubmit bids or make changes until further notice.” - ENS – “Emergency Notice issued due to Market failed, see MIS”. DEVELOP a plan to complete a Day Ahead Ancillary Services market and issue market notifications as needed. |
| 5 | WHEN the Emergency no longer exists, THEN terminate the Emergency Notification: - Make hotline call to QSEs - Make MIS message invalid - Update end time on ENS message |

## 2.22	Restoration of Primary Control Center Functionality

Procedure Purpose:  This procedure is to be performed by Day-Ahead Desk Operator at the Back-up Control Center (BCC) in restoring Day-Ahead Market functions.

- Additional staff has arrived, including a Shift Supervisor,

- BCC has become PCC,

- All EMS applications up and functioning properly

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: May 20, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| NOTE: | Before normal operation can be restored, be sure to communicate with the other desks to determine the current state of the grid and any communications that may have taken place with Market Participants. |
| Prior to Resuming Day-Ahead Market | Upon arrival to the BCC: - Check with the Frequency Control Desk Operator to find out about any calls received from QSEs and if any VDIs were issued, - Check with Operating Period Desk Operator to find out if RTBM deployments have resumed. |
| Market Notification | When you are ready to resume normal operation, place the following Hotline call to the QSEs: - Typical Script: “This call requires everyone to remain on the line until it is complete. [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name], ERCOT is resuming normal, Day-Ahead Market operations, from the back-up control center. Until further notice, direct all Day Ahead Market phone calls to (512) 225-7102.” [Select QSE], “Please repeat that back for me.” - If repeat back is CORRECT, “That is correct, thank you.” If INCORRECT, repeat the entire process until the repeat back is correct. |
| Operator Technical Logs | RECORD the following in the shift log: - Date - Time of event - Description of events, failures, or incident - Actions taken and resolution - Time of system or function restoration |