Power Operations Bulletin #418

ERCOT has revised the Day-Ahead Desk Procedure Manual.

The specific changes are shown below.

A copy of the procedure can be found at:

http://www.ercot.com/mktrules/guides/procedures/index.html

## 2.5	Priority Outages

Procedure Purpose:  To monitor outages that affect generation.

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. | TOP-002-2 R1 & R5 |  |  |  |

| Approved by: |  |  | Effective Date: June 18, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| NOTE: | A Priority Outage is one that affects generation. |
| 1 | Prior to 1600: - From a review of the Outage Notes for the next Operating Day, make lists of Priority Outages, sorted by affected Transmission Operator. - The file is located in P:\DAY AHEAD – TDSP PRIORITY OUTAGE NOTIFICATION - Print copies. - Fax the lists to the respective TOs accompanied by a request that within 30 minutes of the Actual Start and Actual End times of these Priority Outages, they enter the Actual Start and Actual End times in the Outage Scheduler - Place in Daily Log File |

## 2.14	Initiate ERCOT Adjustment Period Replacement Capacity Advisory Study

Procedure Purpose:  Initiate Replacement advisory study in the adjustment period.

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. | EOP-001-0 R3.1 | TOP-002-2 R11 |  |  |

| Approved by: |  |  | Effective Date: June 18, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 2 |  |

| Step | Action |
| --- | --- |
| CAUTION | When an ERCOT Adjustment Period Replacement Capacity STUDY is run, Do NOT select the PUBLISH button on the Study page before you execute the study. Selecting the PUBLISH button on the Study page will OPEN an RPRS Market. |
| 1 | After the close of the Day Ahead Market, as time permits or conditions develop such that additional Replacement Reserve Services may be needed; INITIATE the “Studies” display the appropriate date and intervals to be studied. - Ensure “Execute SFT” is checked. - Ensure “Execute W/L” is checked. - SELECT “Replacement” in the “Study Mode” area. - SELECT the appropriate hours through hour ending 2400 of the day of the study SAVE the study with the appropriate name. |
| 2 | RUN a replacement analysis by clicking the “EXECUTE” button. |
| 3 | Review the results to determine if there is a need to procure additional generation, either for congestion or capacity. Consult with the Shift Supervisor as necessary. |
| 4 | If additional generation is needed, determine if there is sufficient time to open an Adjustment Period Replacement Market or if OOMC will be utilized due to time constraints. |
| 5 | If a time constraint exists, OOMC the generation RPRS selected. |
| 6 | LOG the time the Study was initiated and any problems including any actions taken. |

## 2.21	Monthly Testing of Satellite Phone System

Procedure Purpose:  To ensure ERCOT maintains communication capability via the Satellite Phone System.

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. | COM-001-1 R2 |  |  |  |

| Approved by: |  |  | Effective Date: June 18, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 4.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| NOTE: | On the first weekend of each month, between the hours of 0000 Saturday and 0500 Monday, the Satellite Phone System Conference Bridge will be tested with Transmission Service Providers. As the Shift Supervisor makes the calls to the individual Transmission Service Provider, they will set a time that the ERCOT Operator will call the Satellite Phone System Conference Bridge and establish communication with the appropriate Transmission Service Providers. |
| NOTE: | Use the ERCOT Satellite Phone User Guide located in Section 7 of the Desktop Guide for a list of the Transmission Service Providers that will be contacted by the ERCOT Operator and instructions on how to place a Satellite Phone System Conference Bridge Call. |
| 1 | - On page 11 of the MX Turret phone select the appropriate line that applies to the ERCOT Operations Desk that is making the call. These numbers are located in the bottom right of the page 11 screen. BKST Day- Day Ahead Desk BKST Freq.- Frequency Desk BKST OPD – Operating Period Desk BKST TS #1 – Transmission Desk #1 BKST TS#2 – Transmission Desk #2 - If the preprogrammed number does not function correctly, refer to Section 7 of the Desktop Guide for the appropriate conference number. - When prompted enter the Moderator Pass Code. - If necessary wait on the line for approx. five minutes for the Participants to dial into the Conference Bridge. - As each participant connects to the Conference Bridge record the name of the individual from the Transmission Service Provider making the call and any problems identified with the connection process. |
| 2 | - If one or more of the Transmission Service Providers fails to connect to the Satellite Phone System Conference Bridge Call investigate the cause and log the following: - Reason for inability to connect - Actions taken by ERCOT - Establish a time for a retest of the Transmission Service Providers not able to connect in the initial test. |
| 3 | When testing is completed, fill out the portion of the Monthly Satellite Conference Bridge Testing Form. An electronic copy of this form is located in P:\ SYSTEM OPERATIONS\ ”Satellite Bridge Testing forms” folder. |
| 4 | When the applicable portion of the Monthly Satellite Conerence Bridge Testing Form is completed, save the form and close the file. Notify the Shift Supervisor that the Conference Bridge Call is complete and that the applicable portion of the form is complete. |
| 5 | Log the test date and results in the ERCOT Operations Log. |
| 6 | When working out of the Back-up during the monthly scheduled dates, test at least one Satellite phone. If any problems are encountered, followed step 7. |
| 7 | Verify with the Shift Supervisor that the following personnel are notified for any Satellite phone problems during the test by e-mail: - 1 ERCOT Training Department - 1 ERCOT Shift Supervisors |