## Power Operations Bulletin # 423

ERCOT has revised the Operating Period Desk Procedure Manual.

The Various Changes are shown below.

A copy of the procedure can be found at:

http://www.ercot.com/mktrules/guides/procedures/index.html

## 2.1.2	Respond to Hour-Ahead Study Results

Procedure Purpose:  To determine A/S scheduling shortage in Hour-Ahead

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. | TOP-002-2 R1 |  |  |  |

| Approved by: |  |  | Effective Date: July 15, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 3.0 | Revision: | 3 |  |

| Step | Action |
| --- | --- |
| 1 | REVIEW HA details of the study. |
| 2 | VERIFY URS margin is acceptable (not indicated in red or due to rounding errors) and proceed. IF in red: - Select interval with largest deficiency, right click and select “show output for this interval”  Identify the QSE where deficiency exists, and wait until the Real Time Balancing Market has solved. - If deficiency still exists, operators may contact the QSE to request an updated Resource plan. |
| 3 | VERIFY DRS margin is acceptable (not indicated in red or due to rounding errors) and proceed. IF in red: - Select interval with largest deficiency, right click and select “show output for this interval” - Identify the QSE where deficiency exists, and wait until the Real Time Balancing Market has solved. - If deficiency still exists, operators may contact the QSE to request an updated Resource plan. |
| 4 | VERIFY RRS margin is acceptable (not indicated in red or due to rounding errors) and proceed. IF in red: - Select interval with largest deficiency, right click and select “show output for this interval”  Identify the QSE where deficiency exists, and wait until the Real Time Balancing Market has solved - If deficiency still exists, operators may contact the QSE to request an updated Resource Plan. |
| 5 | - Identifying NSRS margin is not possible using the MOI with the implementation of PRR776. - For NSRS calculations, Resources must also be available, off line and have NSRS checked in the resource plan to be counted in the Excel MAI. |
| NOTE: | If the QSE agrees to correct the deficiency, log and take no further immediate action. If they are unable to correct the deficiency (i.e. Insufficient resources are available to provide agreed upon services), then notify the Shift Supervisor of possible protocol violation(s). |
| NOTE: | Once the maximum procurement level has been reached, increasing the offset will not lead to additional balancing deployment. The maximum procurement level is reached once “Net Gen” and “Net Load” no longer matches in the Real-Time Balancing Market window. Should Balancing Energy become depleted, over compensating with excessive offsets can potentially cause unexpected deployments when more balancing becomes available. |
| NOTE: | Deployment of NSRS is based on the Excel MAI along with a reasonable load forecast. Recall is based on real-time UBES availability or other reliability conditions. |

| 6 | Using the Excel MAI, verify UBES Remaining. - If >20% of available UBES remains undeployed, no further action is necessary. - If <20% of available UBES remains undeployed, verify that UBES is available in real-time before entering the offset. - If >5% of available UBES remains undeployed, no further action is necessary. - If <5% remaining proceed. |
| --- | --- |
| 7 | DETERMINE if NSRS deployment is necessary for balancing support or shortage of physical responsive reserve. IF NSRS is scheduled and available, deploy for a minimum of one hour and continue as needed. If NSRS is necessary and is scheduled and available, communicate with the Shift Supervisor to ensure an active constraint won’t be negatively impacted, then deploy NSRS as scheduled and needed to resolve UBES deficiency. |
| 8 | When NSRS is deployed, notification is automatically made to the Market Participants at the following link: http://mospublic.ercot.com/ercot/jsp/balancing_services_mcp.jsp |
| 9 | Allow NSRS to expire when the percent UBES remaining exceeds the amount of NSRS deployed by 10% or greater. |
| 10 | When NSRS is terminated, EROCT will provide notice via the ENS with the following information: Typical Posting: “At IE [interval], NSRS in [all zones or specify zone] ended. The termination time of the message must be set to the end of the day, i.e. 23:59. |

## 2.3.1	Manage the Real Time Balancing Market

Procedure Purpose:  Verify that the RTBM cleared successfully and that all data and deployments that cleared are valid.

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. | TOP-002-2 R1 |  |  |  |

| Approved by: |  |  | Effective Date: July 15, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 3.0 | Revision: | 1 |  |

| Step | Action |
| --- | --- |
| NOTE: | Ref Display: Real Time Balancing Market Typically go to “windows” on the tool bar and left click Real Time Balancing Market. |
| 1 | CONFIRM that the real-time balancing energy calculation has completed for the periods ending hh:15, hh:30, hh:45, or hh:00 as needed to assure system is functioning properly. |
| 2 | REVIEW the real-time balancing results. If Ldr = Success, and Net Gen = Net Load 50 MW, go to Step 4. |
| 3 | IF the following conditions apply: - Ldr = Failed or - Real Time Balancing Market does not approve, or - If the difference between Net Gen and Net Load is >50 MW Notify the Shift Supervisor and Frequency Control Desk to take action as necessary to maintain system reliability. |
| 4 | The operator may verify the generation scheduled in real time balancing matches the generation from the Hour Ahead Study. If the difference is less than +/-100 MWs, determine if new study needs to be run. |
| 5 | IF the ‘Price Admin’ box is checked notify Shift Supervisor and post message on ENS. Typical Message: “The MCPE for interval <interval> on <day and date> has been deemed by ERCOT to be questionable. Appropriate ERCOT personnel will review the posted value.” |

## 2.3.2	Held Intervals

Procedure Purpose:  To notify ERCOT personnel of held real-time balancing market deployments.

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. |  |  |  |  |

| Approved by: |  |  | Effective Date: July 15, 2009 |  |
| --- | --- | --- | --- | --- |
| Version: | 3.0 | Revision: | 0 |  |

| Step | Action |
| --- | --- |
| 1 | Prepare email with the following information: - Last good interval - Interval held - Resumed interval - Date - Number of intervals held - Reason(s) why interval was held if known Example: ERCOT held the 10:00 interval through the 10:15 interval, resumed with the 10:30 interval on 08/01/09. (1 interval) RTB failed to execute. |
| 2 | LOG in the Operator’s Log the held interval information and reason(s) why interval was held. |
| 3 | Send information in an email to the following distribution list: - HELD INTERVAL |