## Power Operations Bulletin # 470

ERCOT has revised the Frequency Control Desk Procedure Manual.

The Various Changes are shown below.

A copy of the procedure can be found at:

http://www.ercot.com/mktrules/guides/procedures/index.html

## 2.1.1 	Capacity and Reserve Monitoring

Procedure Purpose: To ensure the required amount of operating reserves are available to meet the load forecast.

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. | BAL-002-0 R2.4 | IRO-005-2 R1.4, R1.5, R4 |  |  |

| Approved by: |  |  | Effective Date: March 31, 2010 |  |
| --- | --- | --- | --- | --- |
| Version: | 4 | Revision | 4 |  |

| Step | Action |
| --- | --- |
| 1 | Monitor the current ERCOT System Load and Short Term Load Forecast. Reference Displays: Generation Area Status Short-Term Load Forecast Excel MAI (Market Analysis Interface) |
| 2 | Determine that the current ERCOT System generation capacity is sufficient to meet the Short-Term Load Forecast requirements. Reference Displays: Generation Area Status Excel MAI (Market Analysis Interface) |
| 2a | Compare the current generation and load with the planned capacity values from resource plan at least every other hour. Reference Displays: Generation Area Status Daily Operation Plan – Resource Plan Excel MAI (Market Analysis Interface) |
| 2b | When QSE’s representing WGR’s notify ERCOT of possible icing, instruct them to reflect that in their resource plans. |
| 2c | If real-time generation is approaching planned capacity a possible reserve capacity shortage is developing. Discuss situation with Operating Period Desk and Shift Supervisor, and increase monitoring of reserves. Reference Displays: Generation Reserve Status – Available Details Real Time Balancing Market – Balance Bid Tab Real Time Balancing Market – QSE Tab Excel MAI (Market Analysis Interface) |
| 3 | If a projected reserve capacity shortage is developing in the next hour, consider OOMC or deploying of Non-Spinning Reserve Service. Consult with the Operating Period Desk regarding hour-ahead study results. |
| 4 | If OOMC is decided upon, request the Transmission & Security Desk perform the actual instruction. If NSRS is decided upon, request the Operating Period Desk perform the actual instruction. |
| 5 | MONITOR additional available generation capacity and available DC tie capacity. Evaluate amount of additional capacity available in a short time frame using telephone communications to QSEs as appropriate. Reference Displays: Generation Area Status Generation Reserve Status Generation Tie-Line Status |
| 6 | Monitor Responsive Reserves on the QSE Reserve Status Display. If Adjusted Responsive Reserve plus LaaRs is correctly calculated, and is below 2300 MW, - notify the Shift Supervisor - implement EEA - log in the Operator’s Log any decision to declare an EEA |
| 7 | Monitor Ancillary Service Obligations. If a QSE is not meeting its Ancillary Services obligations as shown on the QSE Reserve Status Display, - contact the QSE to correct the deficiency - notify the Shift Supervisor of the QSE’s response - log the activities performed in connection with this step |
| NOTE: | Due to generation minimum down time, if NSRS is deployed and has ended, it would be acceptable for a QSE to keep their unit(s) providing non-spin on-line at its LSL. If NSRS is deployed again, it would need to move to its HSL. |

## 2.2.3 	Testing of Satellite Phone and NXT Communicator

Procedure Purpose: To ensure ERCOT maintains communication capability via the Satellite Phone System and NXT Communicator.

| Protocol Ref. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Ref. |  |  |  |  |
| NERC Std. | COM-001-1.1 R2 | EOP-005-1 R5 |  |  |

| Approved by: |  |  | Effective Date: March 31, 2010 |  |
| --- | --- | --- | --- | --- |
| Version: | 4 | Revision: | 4 |  |

| Step | Action |
| --- | --- |
|  | Primary Control Center |
| NOTE: | When a participant dials into the conference bridge before the moderator dials in, they will hear music and be placed on hold. |
| NOTE: | On the first weekend of each month, between the hours of 0000 Saturday and 0500 Monday, the Satellite Phone System Conference Bridge will be tested with Transmission Service Providers. As the Shift Supervisor makes the calls to the individual Transmission Service Provider, they will set a time that the ERCOT Operator will call the Satellite Phone System Conference Bridge and establish communication with the appropriate Transmission Service Providers. |
| NOTE: | Use the ERCOT Satellite Phone User Guide located in Section 7 of the Desktop Guide for a list of the Transmission Service Providers that will be contacted by the ERCOT Operator and instructions on how to place a Satellite Phone System Conference Bridge Call. |
| 1 | - On page 11 of the MX Turret phone select the appropriate line that applies to the ERCOT Operations Desk that is making the call. These numbers are located in the bottom right of the page 11 screen. BKST Day- Day Ahead Desk BKST Freq.- Frequency Desk BKST OPD – Operating Period Desk BKST TS #1 – Transmission Desk #1 BKST TS#2 – Transmission Desk #2 - If the preprogrammed number does not function correctly, refer to Section 7 of the Desktop Guide for the appropriate conference number. - When prompted enter the Moderator Pass Code. - If necessary wait on the line for approx. five minutes for the Participants to dial into the Conference Bridge. - As each participant connects to the Conference Bridge record the name of the individual from the Transmission Service Provider making the call and any problems identified with the connection process. |
| 2 | - If one or more of the Transmission Service Providers fails to connect to the Satellite Phone System Conference Bridge Call investigate the cause and log the following: - Reason for inability to connect - Actions taken by ERCOT - Establish a time for a retest of the Transmission Service Providers not able to connect in the initial test. |
| 3 | Inform the Shift Supervisor when test is complete indicating any issues identified. |
| 4 | Log the test results in the Operations Log. |
|  | Alternate Control Center |
| 1 | When working out of the alternate control center during the monthly scheduled dates: - Test the Satellite phone - Test NXT Communicator by activating the “Test Message Monthly”. Refer to Desktop Guide 7.10 (Test early evening). If problems are encountered with NXT Communicator, open help ticket. |
| 2 | Inform the Shift Supervisor when test is complete indicating any issues identified. |
| 3 | Log the test results in the Operations Log. |