- Power Operations Bulletin # 1101

ERCOT has posted/revised the Scripts manual.

The Various Changes are shown below.

A copy of the procedure can be found at:

- http://www.ercot.com/mktrules/guides/procedures/index.html

## Q#7 EEA2 Media Appeal Only:

“This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name], at [xx:xx], ERCOT declared EEA 2 and is issuing an appeal through the public news media for voluntary energy conservation.  Implement all measures associated with EEA 1 if not already implemented.  [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## Q#55 Transmission Watch for Post-Contingency Overload:

“This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is issuing a Transmission Watch due to the post-contingency overload of [geographical area ], [mitigation plan/temporary outage action plan is being developed] [manual actions are being performed to reduce post-contingency overload].  [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## Q#56 Cancel Transmission Watch for Post-Contingency Overload:

“This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is canceling the Transmission Watch for post-contingency overload of [geographical area ].  [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## T#34 Transmission Watch for Post-Contingency Overload:

“This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is issuing a Transmission Watch due to the post-contingency overload in the [geographical area], [mitigation plan/temporary outage action plan is being developed] [manual actions are being performed to reduce post-contingency overload].  [TO] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## T#35 Cancel Transmission Watch for Post-Contingency Overload:

“This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is canceling the Transmission Watch for post-contingency overload of [geographical area ] a [mitigation plan/temporary outage action plan] has been developed.  [TO] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

Preparation

| Prepared by | Role | Date Completed |  |
| --- | --- | --- | --- |
| Frosch and Hartmann | Prepares | April 23,2015 |  |
| Frosch, Hartmann, and Gaddy | Procedure writers and editors | July 13, 2015 |  |
| Hartmann and Gaddy | Procedure writers and editors | September 25, 2015 |  |
| Hartmann, Gaddy, and Drummond-Bunner | Procedure writers and editors | December 22, 2015 |  |
| Hartmann and Gaddy | Procedure writers and editors | January 22, 2016 |  |
| Hartmann and Gaddy | Procedure writers and editors | February 23, 2016 |  |
| Hartmann and Gaddy | Procedure writers and editors | April 22, 2016 |  |
| Frosch, Hartmann, and Gaddy | Procedure writers and editors | June 24, 2016 |  |
| Frosch, Hartmann, and Gaddy | Procedure writers and editors | September 26, 2016 |  |
| Hartmann and Gaddy | Procedure writers and editors | October 31, 2016 |  |
| Hartmann and Gaddy | Procedure writers and editors | December 22, 2016 |  |
| Hartmann, Gaddy, and Frosch | Procedure writers and editors | March 27, 2017 |  |
| Hartmann and Gaddy | Procedure writers and editors | April 5, 2017 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | May 26, 2017 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | June 23, 2017 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | August 30, 2017 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | September 27, 2017 |  |
| Hartmann, Gaddy, Ballew, and Frosch | Procedure writers and editors | February 22, 2018 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | March 26, 2018 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | April 27, 218 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | August 24, 2018 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | September 27, 2018 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | October 29, 2018 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | December 21, 2018 |  |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | January 25, 2019 |  |
| Hartmann, Gaddy, and Luker | Procedure writers and editors | April 26, 2019 |  |
| Hartmann, Gaddy, and Pence | Procedure writers and editors | May 13, 2019 |  |
| Hartmann, Gaddy, and Pence | Procedure writers and editors | May 28, 2019 |  |
| Hartmann, Gaddy, and Pence | Procedure writers and editors | July 29, 2019 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | October 29, 2019 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | December 20, 2019 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | January 27, 2020 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | May 26, 2020 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | June 25, 2020 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | September 23, 2020 |  |
| Hartmann, Gaddy, Cyphers, and Sheets | Procedure writers and editors | December 23, 2020 |  |
| Hartmann, Gaddy, Cyphers, and Sheets | Procedure writers and editors | March 26, 2021 |  |
| Hartmann, Gaddy, Cyphers, and Sheets | Procedure writers and editors | July 28, 2021 |  |
| Hartmann, Gaddy, Cyphers and Sheets | Procedure writers and editors | August 27, 2021 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | November 29, 2021 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | December 1, 2021 |  |
| Hartmann, Gaddy, Cyphers, and Frosch | Procedure writers and editors | December 17, 2021 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | January 27, 2022 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | May 20, 2022 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | May 20, 2022 |  |
| Hartmann, Gaddy, Cyphers, and Frosch | Procedure writers and editors | July 25, 2022 |  |
| Hartmann, Gaddy, Cyphers, and Frosch | Procedure writers and editors | December 22, 2022 |  |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | January 26, 2023 |  |
| Hartmann, Gaddy, Cyphers, and Frosch | Procedure writers and editors | March 27, 2023 |  |
| Hartmann, Gaddy, Cyphers, and Luker | Procedure writers and editors | April 25, 2023 |  |
| Hartmann, Gaddy, Cyphers, and Luker | Procedure writers and editors | June 5, 2023 |  |
| Hartmann, Gaddy, Cyphers, and Luker | Procedure writers and editors | June 26, 2023 |  |
| Hartmann, Gaddy, Cyphers, and Luker | Procedure writers and editors | August 29, 2023 |  |
| Hartmann, Cyphers, and Luker | Procedure writers and editors | September 28, 2023 |  |

Manual Change History

| Procedure | Ver/Rev | Reason for Issue | Effective Date |  |
| --- | --- | --- | --- | --- |
| All Sections | 1.0 / 0 | New script document | May 1, 2015 |  |
| All Sections 1 | 1.0 / 1 1.0 / 1 | Added a “Q” for QSE scripts and a “T” for TO scripts Updated Scripts Q62, Q63, Q64 | July 15, 2015 |  |
| 3 | 1.0 / 2 | Updated Script T#8 Content Updated Script T#29 Content and Script Title Corrected Script T#71 Content and Script titles | October 1,2015 |  |
| 2 3 | 1.0 / 2 1.0 / 3 | Updated QSE script titles Q#1, Q#2, Q#3, Q#4, Q#5, Q#6, Q#7, Q#8, Q#9, Q#12, Q#14, Q#15, Q#16, Q#22, Q#23, Q#26, Q#27, Q#28, Q#29, Q#30, Q#31, Q#32, Q#33, Q#34, Q#35, Q#36, Q#38, Q#39, Q#40, Q#41, Q#42, Q#43, Q#44, Q#46, Q#47, Q#48, Q#49, Q#50, Q#51, Q#52, Q#53, Q#54, Q#55, Q#56, Q#57, Q#61, Q#65, Q#69, Q#70, Q#71, Q#72, Q#73, Q#74, Q#75, Q#78, Q#79, Q#80, Q#81, Q#82, Q#84, Q#86, Q#87, Q#88, Q#91, Q#92, Q#94, Q#95, Q#96, Q#97, Q#99, Q#100, & Q#101 Deleted scripts Q#71 & Q#72 Updated Transmission script titles T#2, T#4, T#9, T#10, T#13, T#14, T#18, T#19, T#20, T#21, T#22, T#23, T#24, T#25, T#26, T#27, T#28, T#29, T#30, T#31, T#32, T#33, T#34, T#35, T#36, T#37, T#38, T#39, T#40, T#41, T#42, T#43, T#44, T#45, T#46, T#47, T#48, T#51, T#52, T#53, T#54, T#55, T#56, T#57, T#58, T#59, T#60, T#61, T#62, T#63, T#64, T#65, T#66, T#67, T#68, T#69, T#70, T#71, T#75, T#76, T#77, T#78, T#79, T#80, T#81, T#82, T#83, T#84, & T#85 Updated script content T#7, T#15, T#22, & T#24 | December 31, 2015 |  |
| 2 3 | 1.0 / 3 1.0 / 4 | Added Script Q#72 Updated Script Titles Q#75, Q#77, Q#78, Q#81, Q#83, & Q#84 Updated Script T#52 & T#85 Added Script T#86 | February 10, 2016 |  |
| 2 3 | 1.0 / 4 - / 5 | Updated Script Q#24, Q#25 and Q#75 Updated Script Title Q#72 Updated Script T#85 Updated Script Title T#86 | March 1, 2016 |  |
| 2 3 | 1.0 / 5 1.0 / 6 | Updated Script Q#19 and Q#21 Updated Script Title Q#21 Updated Script T#16 and T#18 Updated Script Title T#18 | April 29, 2016 |  |
| 1 2 3 | 1.0 / 2 1.0 / 6 1.0 / 7 | Updated 1.2 Updated Scripts Q#40, Q#41, Q#52, Q#53, Q#59, Q#68, Q#69 & added Script Q#102 Updated Scripts T#8, T#9, T#10, T#28, T#79, T#82, T#83 & added Script T#87 | June 30, 2016 |  |
| 2 3 | 1.0 / 7 1.0 / 8 | Updated scripts Q#20, Q#32, Q#33, Q#34, Q#35, & Q#69 Updated scripts T#21, T#23, T#41, T#43 T#44, T#61, & T#62 | September 30, 2016 |  |
| 2 | 1.0 / 8 | Added script Q#71 | November 2, 2016 |  |
|  |  | Updated Table of Contents Links | December 30, 2016 |  |
| 2 3 | 1.0 / 9 1.0 / 9 | Updated script Q#11 and Q#71 Added script Q#103 Updated scripts T#8, T#25, T#26 and T#29 Added script T#88 | March 31, 2017 |  |
| 2 3 | 1.0 / 10 1.0 / 10 | Added scripts Q#104, Q#105, Q#106, Q#107, Q#108 and Q#109 Added scripts T#89, T#90, T#91, T#92, T#93, T#94, T#95, T#96, T#97 and T#98 | April 6, 2017 |  |
| 2 3 | 1.0 / 11 1.0 / 11 | Updated script Q#106 Updated script T#91 | June 1, 2017 |  |
| 2 3 | 1.0 / 12 1.0 / 12 | Deleted script Q#102 Deleted script T#87 Updated script T#91 | June 30, 2017 |  |
| 2 3 | 1.0 / 13 1.0 / 13 | Added scripts Q#110 and Q#111 Updated script T#63 and T#92 | August 31, 2017 |  |
| 2 3 | 1.0 / 14 1.0 / 14 | Updated script Q#22, Q#50, Q#89, Q#91, Q#93, Q#94, Q#95, & Q#107 Updated script T#23 | September 29, 2017 |  |
| 2 3 | 1.0 / 15 1.0 / 15 | Updated script Q108 & 109 Updated script T6, T97 & T98 | February 28, 2018 |  |
| 3 | 1.0 / 16 | Updated scripts T25 and T26 | March 30, 2018 |  |
| 2 3 | 1.0 / 16 1.0 / 17 | Added scripts Q112 through Q119 Updated script T17 Added scripts T99 & T100 | May 1, 2018 |  |
| 2 | 1.0 / 17 | Added scripts Q#120 & Q#121 | August 31, 2018 |  |
| 2 3 | 1.0 / 18 1.0 / 18 | Deleted script #Q106 & Q#107 for NPRR 825 Deleted script #T91 & T#92 for NPRR 825 | October 1, 2018 |  |
| 2 3 | 1.0 / 19 1.0 / 19 | Updated script Q#18 & Q#21 Updated script T#18 | November 1, 2018 |  |
| 2 3 | 1.0 / 20 1.0 / 20 | Update scripts Q#32, Q#48, Q#49 & Q#71 Updated script T#22 & T#29 | December 28, 2018 |  |
| 2 | 1.0 / 21 | Updated script Q#100 | January 31, 2019 |  |
| 2 | 1.0 / 22 | Updated scripts Q#6, Q#7, Q#9 & Q#12 | May 1, 2019 |  |
| 2 | 1.0 / 23 | Added scripts Q#122, Q#123, and Q#124 | May 16, 2019 |  |
| 2 3 | 1.0 / 24 1.0 / 21 | Updated script Q#124 Updated script title T#6 | May 31, 2019 |  |
| 2 3 | 1.0 / 25 1.0 / 22 | Deleted Script Q#26 Added Scripts Q#106 & Q#107 Deleted Scripts T#39 & T#40 Added Scripts T#91 & T#92 | August 1, 2019 |  |
| 2 3 | 1.0 / 25 1.0 / 23 | Updated scripts Q#6, Q#7, Q#8, Q#9, Q#26, Q#58, Q#59, Q#62, Q#63, Q#66, Q#67, Q#75, Q#122, Q#123 & Q#124 and added scripts Q#26 & Q#125 Added script T#39 and updated scripts T#72, T#73 & T#85 | November 1, 2019 |  |
| 2 3 | 1.0 / 27 1.0 / 24 | Updated script Q#103, Q#117 & Q#122 Updated script T#88 | January 1, 2019 |  |
| 2 3 | 1.0 / 28 1.0 / 25 | Updated script Q#89 & Q#110 Updated Script T#63 | January 31, 2020 |  |
| 3 | 1.0 / 26 | Updated Script T#53 | May 29, 2020 |  |
| 2 3 | 1.0 / 29 1.0 / 27 | Added Scripts Q#126, Q#127 & Q#128 per NPRR930 Added Scripts T#101, T#102 & T#103 per NPRR930 | July 1, 2020 |  |
| 2 | 1.0 / 30 | Updated script Q#123 & Q#124 | October 1, 2020 |  |
| 2 3 | 1.0 / 31 1.0 / 28 | Updated Scripts Q#108 & Q#109 Updated Scripts T#97 & T#98 | December 31, 2020 |  |
| 2 | 1.0 / 32 | Updated Script Q#13 | April 1, 2021 |  |
| 2 3 | 1.0 / 33 1.0 / 29 | Deleted Q#110 & Q#111 Deleted T#63 & T#64 | July 31, 2021 |  |
| 2 3 | 1.0 / 34 1.0 / 30 | Updated Script Q#75 Updated Script T#85 | September 1, 2021 |  |
| 2 3 | 1.0 / 35 1.0 / 31 | Updated Script Q#126 Added Scripts Q#102, Q#109, Q#110,Q#129 & Q#130 Added Script T#40 | December 1, 2021 |  |
| 3 | 1.0 / 32 | Updated Script T#40 | December 17, 2021 |  |
| 2 3 | 1.0 / 36 1.0 / 33 | Updated Scripts Q#15, Q#62, Q#63, Q#64, Q#102 & Q#111 Updated Script T#12 All scripts in this manual have been reviewed | December 31, 2021 |  |
| 3 | 1.0 / 34 | Updated Scripts T#1, T#7, T#76, T#77, T#78 & deleted script T#40 | February 1, 2022 |  |
| 2 3 | 1.0 / 37 1.0 / 35 | Updated Script Q#8 Added Scrip Q#131 Added Script T#40 | May 26, 2022 |  |
| 2 3 | 1.0 / 38 1.0 / 36 | Updated Script Q#1, Q#2, Q#21, Q#110, & Q#111 Updated Script T#2 | July 29, 2022 |  |
| 2 3 | 1.0 / 39 1.0 / 36 | Added and updated scripts Q#5, Q#6, Q#7, Q#9, Q#64, Q#110, Q#111, Q132, & Q133 Added and updated scripts T#63, T#64, T#78, & T#87 All scripts in this manual have been reviewed | December 30, 2022 |  |
| 2 3 | 1.0 / 40 1.0 / 37 | Updated script Q#11 Updated script T#8 | February 1, 2023 |  |
| 2 3 | 1.0 / 41 1.0 / 38 | Q#62, Q#63, Q#64, Q#65, Q#66, Q#67, Q#68, Q#69, & Q#70 T#84 | March 31, 2023 |  |
| 2 | 1.0 / 42 | Updated script Q#126 | April 28, 2023 |  |
| 2 | 1.0 / 43 | Updated scripts Q#8, Q#26, Q#39, Q#42, Q#130, & Q#131 | June 9, 2023 |  |
| 2 3 | - / 44 1.0 / 39 | Updated scripts Q#5, Q#83 Updated scripts T#67, T#87 | June 30, 2023 |  |
| 2 3 | 1.0 / 45 1.0 / 40 | Updated script Q#10, Q#55 Updated Script T#34, T#39 | September 1, 2023 |  |
| 2 3 | 1.0 / 46 1.0 / 41 | Updated Script Q#7 Updated Scrpt T#34 | October 4, 2023 |  |