- Power Operations Bulletin # 1107

ERCOT has posted/revised the Scripts manual.

The Various Changes are shown below.

A copy of the procedure can be found at:

- http://www.ercot.com/mktrules/guides/procedures/index.html

Q#1 Watch for PRC <3000 MW Deployed ERS-30 / ERS-10 / both ERS-30 and ERS-10 / Weather Sensitive:

“This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is issuing an Watch due to Physical Responsive Capability falling below 3000 MW. [All 10 min / 30 min / both 10 min and 30 min including Weather Sensitive ERS Resources with an obligation have been deployed. Distribution voltage reduction has been requested.] Suspend all Resource testing and update telemetered Resource Status Codes.  ERCOT is reminding you, that you can schedule available interchange into ERCOT across the DC Ties. [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## Q#2 Cancel Watch for PRC <3000 MW Recalling ERS-10 / ERS-30 / both  ERS-10 and ERS-30 / Weather Sensitive:

“This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is canceling the Watch for Physical Responsive Capability.  Physical Responsive Capability is now above 3000 MW. [ERCOT is recalling all 10 min / 30 min / both 10 min and 30 min including Weather Sensitive.  Distribution voltage reduction has returned to normal operation.] Approved Resource testing may resume as scheduled. [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

Q#3 Blank:

Q#4 Blank:

.

## Q#9 Watch/EEA 2 / 3 deployed ERS-10 / ERS-30 / both ERS-10 and ERS-30 / ERS-10, ERS-30 including Weather Sensitive New Time Period Script:

“This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name], at [xx:xx], ERCOT is in [Watch/EEA [2 / 3].  All [10 min / 30 min / both the 10 min and 30 min / 10 min and 30 min including Weather Sensitive] ERS Resources with an obligation have been deployed for new Time Period [X].    [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## Q#11 EEA3  Firm Load Shed:

“This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name], at [xx:xx], ERCOT is declaring EEA 3 with firm load shed, [MW Amount] of firm load is being shed from the [winter season/summer season] load shed table].  Implement all measures associated with EEA 1 and 2, if not already implemented. [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## Q#12 Blank

## Q#17 Cancel Watch for PRC below <3000:

“This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is canceling the Watch for Physical Responsive Capability.   At this time, Resource testing may resume.  [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## Q#103 Blank

## T#1 Watch for PRC <3000 MW:

“This call requires everyone to remain on the line until it is complete.  [TO] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is issuing an Watch due to Physical Responsive Capability falling below 3000 MW. TO’s are instructed to reduce customer loads by using distribution voltage reduction measures if available. ERS has been deployed. [TO] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## T#2 Cancel Watch for PRC <3000 MW:

“This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is canceling the Watch for Physical Responsive Capability.  Physical Responsive Capability is now above 3000 MW and all ERS has been recalled. TO’s that utilized distribution voltage reduction measures can return to normal operation.  [TO] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

## T#3 Blank (Future):

## T#4 Blank (Future):

## T#5 EEA1:

“This call requires everyone to remain on the line until it is complete.  [TO] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name]; at [xx:xx], ERCOT is declaring EEA 1 due to due to Physical Responsive Capability falling below 2500 MW. [TO] please repeat this back to me.  That is correct, thank you”

## T#6Blank:

## T#7 EEA2 Media Appeal and Voltage Reduction:

“This call requires everyone to remain on the line until it is complete.  [TO] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name], at [xx:xx], ERCOT is declaring EEA 2 due to [Physical Responsive Capability falling below 2000 MW] [the clock-minute average system frequency is below 59.91 Hz for 15 consecutive minutes] [steady-state frequency falls below 59.8 Hz] and is issuing an appeal through the public news media for voluntary energy conservation.  Reduce customer loads by using distribution voltage reduction measures, if deemed beneficial [and any available capacity from Load Management Programs if available,] [Load Resources and / or ERS have been deployed].

[TO] please repeat this back to me.  That is correct, thank you”

## T#8 EEA3 Firm Load Shed:

“This call requires everyone to remain on the line until it is complete.  [TO] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name], at [xx:xx], ERCOT is declaring EEA 3 firm load shed due to [Physical Responsive Capability falling below 1500 MW] [the clock-minute average system frequency is below 59.91 Hz for 20 consecutive minutes] [steady-state frequency falls below 59.8 Hz].  ERCOT is issuing an Operating Instruction for all Transmission Operators to shed their share of [amount] MW from the [winter season/summer season] load shed table].  Transmission Operators are to report to ERCOT when this is complete.  Implement all measures associated with EEA 1 and 2, if not already implemented. [TO] please repeat this back to me.  That is correct, thank you”.

## T#14 Cancel Watch for PRC<3000 MW:

“This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is canceling the Watch for Physical Responsive Capability.  Physical Responsive Capability is now above 3000 MW.  [TO] please repeat this back to me.  That is correct, thank you.”

## T#88 Blank :

Document Control

Preparation

| Prepared by | Role | Date Completed |
| --- | --- | --- |
| Frosch and Hartmann | Prepares | April 23,2015 |
| Frosch, Hartmann, and Gaddy | Procedure writers and editors | July 13, 2015 |
| Hartmann and Gaddy | Procedure writers and editors | September 25, 2015 |
| Hartmann, Gaddy, and Drummond-Bunner | Procedure writers and editors | December 22, 2015 |
| Hartmann and Gaddy | Procedure writers and editors | January 22, 2016 |
| Hartmann and Gaddy | Procedure writers and editors | February 23, 2016 |
| Hartmann and Gaddy | Procedure writers and editors | April 22, 2016 |
| Frosch, Hartmann, and Gaddy | Procedure writers and editors | June 24, 2016 |
| Frosch, Hartmann, and Gaddy | Procedure writers and editors | September 26, 2016 |
| Hartmann and Gaddy | Procedure writers and editors | October 31, 2016 |
| Hartmann and Gaddy | Procedure writers and editors | December 22, 2016 |
| Hartmann, Gaddy, and Frosch | Procedure writers and editors | March 27, 2017 |
| Hartmann and Gaddy | Procedure writers and editors | April 5, 2017 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | May 26, 2017 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | June 23, 2017 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | August 30, 2017 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | September 27, 2017 |
| Hartmann, Gaddy, Ballew, and Frosch | Procedure writers and editors | February 22, 2018 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | March 26, 2018 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | April 27, 218 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | August 24, 2018 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | September 27, 2018 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | October 29, 2018 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | December 21, 2018 |
| Hartmann, Gaddy, and Ballew | Procedure writers and editors | January 25, 2019 |
| Hartmann, Gaddy, and Luker | Procedure writers and editors | April 26, 2019 |
| Hartmann, Gaddy, and Pence | Procedure writers and editors | May 13, 2019 |
| Hartmann, Gaddy, and Pence | Procedure writers and editors | May 28, 2019 |
| Hartmann, Gaddy, and Pence | Procedure writers and editors | July 29, 2019 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | October 29, 2019 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | December 20, 2019 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | January 27, 2020 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | May 26, 2020 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | June 25, 2020 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | September 23, 2020 |
| Hartmann, Gaddy, Cyphers, and Sheets | Procedure writers and editors | December 23, 2020 |
| Hartmann, Gaddy, Cyphers, and Sheets | Procedure writers and editors | March 26, 2021 |
| Hartmann, Gaddy, Cyphers, and Sheets | Procedure writers and editors | July 28, 2021 |
| Hartmann, Gaddy, Cyphers and Sheets | Procedure writers and editors | August 27, 2021 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | November 29, 2021 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | December 1, 2021 |
| Hartmann, Gaddy, Cyphers, and Frosch | Procedure writers and editors | December 17, 2021 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | January 27, 2022 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | May 20, 2022 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | May 20, 2022 |
| Hartmann, Gaddy, Cyphers, and Frosch | Procedure writers and editors | July 25, 2022 |
| Hartmann, Gaddy, Cyphers, and Frosch | Procedure writers and editors | December 22, 2022 |
| Hartmann, Gaddy, and Cyphers | Procedure writers and editors | January 26, 2023 |
| Hartmann, Gaddy, Cyphers, and Frosch | Procedure writers and editors | March 27, 2023 |
| Hartmann, Gaddy, Cyphers, and Luker | Procedure writers and editors | April 25, 2023 |
| Hartmann, Gaddy, Cyphers, and Luker | Procedure writers and editors | June 5, 2023 |
| Hartmann, Gaddy, Cyphers, and Luker | Procedure writers and editors | June 26, 2023 |
| Hartmann, Gaddy, Cyphers, and Luker | Procedure writers and editors | August 29, 2023 |
| Hartmann, Cyphers, and Luker | Procedure writers and editors | September 28, 2023 |
| Hartmann, Cyphers, and Luker | Procedure writers and editors | October 26, 2023 |

Manual Change History

| Procedure | Ver/Rev | Reason for Issue | Effective Date |
| --- | --- | --- | --- |
| All Sections | 1.0 / 0 | New script document | May 1, 2015 |
| All Sections 1 | 1.0 / 1 1.0 / 1 | Added a “Q” for QSE scripts and a “T” for TO scripts Updated Scripts Q62, Q63, Q64 | July 15, 2015 |
| 3 | 1.0 / 2 | Updated Script T#8 Content Updated Script T#29 Content and Script Title Corrected Script T#71 Content and Script titles | October 1,2015 |
| 2 3 | 1.0 / 2 1.0 / 3 | Updated QSE script titles Q#1, Q#2, Q#3, Q#4, Q#5, Q#6, Q#7, Q#8, Q#9, Q#12, Q#14, Q#15, Q#16, Q#22, Q#23, Q#26, Q#27, Q#28, Q#29, Q#30, Q#31, Q#32, Q#33, Q#34, Q#35, Q#36, Q#38, Q#39, Q#40, Q#41, Q#42, Q#43, Q#44, Q#46, Q#47, Q#48, Q#49, Q#50, Q#51, Q#52, Q#53, Q#54, Q#55, Q#56, Q#57, Q#61, Q#65, Q#69, Q#70, Q#71, Q#72, Q#73, Q#74, Q#75, Q#78, Q#79, Q#80, Q#81, Q#82, Q#84, Q#86, Q#87, Q#88, Q#91, Q#92, Q#94, Q#95, Q#96, Q#97, Q#99, Q#100, & Q#101 Deleted scripts Q#71 & Q#72 Updated Transmission script titles T#2, T#4, T#9, T#10, T#13, T#14, T#18, T#19, T#20, T#21, T#22, T#23, T#24, T#25, T#26, T#27, T#28, T#29, T#30, T#31, T#32, T#33, T#34, T#35, T#36, T#37, T#38, T#39, T#40, T#41, T#42, T#43, T#44, T#45, T#46, T#47, T#48, T#51, T#52, T#53, T#54, T#55, T#56, T#57, T#58, T#59, T#60, T#61, T#62, T#63, T#64, T#65, T#66, T#67, T#68, T#69, T#70, T#71, T#75, T#76, T#77, T#78, T#79, T#80, T#81, T#82, T#83, T#84, & T#85 Updated script content T#7, T#15, T#22, & T#24 | December 31, 2015 |
| 2 3 | 1.0 / 3 1.0 / 4 | Added Script Q#72 Updated Script Titles Q#75, Q#77, Q#78, Q#81, Q#83, & Q#84 Updated Script T#52 & T#85 Added Script T#86 | February 10, 2016 |
| 2 3 | 1.0 / 4 - / 5 | Updated Script Q#24, Q#25 and Q#75 Updated Script Title Q#72 Updated Script T#85 Updated Script Title T#86 | March 1, 2016 |
| 2 3 | 1.0 / 5 1.0 / 6 | Updated Script Q#19 and Q#21 Updated Script Title Q#21 Updated Script T#16 and T#18 Updated Script Title T#18 | April 29, 2016 |
| 1 2 3 | 1.0 / 2 1.0 / 6 1.0 / 7 | Updated 1.2 Updated Scripts Q#40, Q#41, Q#52, Q#53, Q#59, Q#68, Q#69 & added Script Q#102 Updated Scripts T#8, T#9, T#10, T#28, T#79, T#82, T#83 & added Script T#87 | June 30, 2016 |
| 2 3 | 1.0 / 7 1.0 / 8 | Updated scripts Q#20, Q#32, Q#33, Q#34, Q#35, & Q#69 Updated scripts T#21, T#23, T#41, T#43 T#44, T#61, & T#62 | September 30, 2016 |
| 2 | 1.0 / 8 | Added script Q#71 | November 2, 2016 |
|  |  | Updated Table of Contents Links | December 30, 2016 |
| 2 3 | 1.0 / 9 1.0 / 9 | Updated script Q#11 and Q#71 Added script Q#103 Updated scripts T#8, T#25, T#26 and T#29 Added script T#88 | March 31, 2017 |
| 2 3 | 1.0 / 10 1.0 / 10 | Added scripts Q#104, Q#105, Q#106, Q#107, Q#108 and Q#109 Added scripts T#89, T#90, T#91, T#92, T#93, T#94, T#95, T#96, T#97 and T#98 | April 6, 2017 |
| 2 3 | 1.0 / 11 1.0 / 11 | Updated script Q#106 Updated script T#91 | June 1, 2017 |
| 2 3 | 1.0 / 12 1.0 / 12 | Deleted script Q#102 Deleted script T#87 Updated script T#91 | June 30, 2017 |
| 2 3 | 1.0 / 13 1.0 / 13 | Added scripts Q#110 and Q#111 Updated script T#63 and T#92 | August 31, 2017 |
| 2 3 | 1.0 / 14 1.0 / 14 | Updated script Q#22, Q#50, Q#89, Q#91, Q#93, Q#94, Q#95, & Q#107 Updated script T#23 | September 29, 2017 |
| 2 3 | 1.0 / 15 1.0 / 15 | Updated script Q108 & 109 Updated script T6, T97 & T98 | February 28, 2018 |
| 3 | 1.0 / 16 | Updated scripts T25 and T26 | March 30, 2018 |
| 2 3 | 1.0 / 16 1.0 / 17 | Added scripts Q112 through Q119 Updated script T17 Added scripts T99 & T100 | May 1, 2018 |
| 2 | 1.0 / 17 | Added scripts Q#120 & Q#121 | August 31, 2018 |
| 2 3 | 1.0 / 18 1.0 / 18 | Deleted script #Q106 & Q#107 for NPRR 825 Deleted script #T91 & T#92 for NPRR 825 | October 1, 2018 |
| 2 3 | 1.0 / 19 1.0 / 19 | Updated script Q#18 & Q#21 Updated script T#18 | November 1, 2018 |
| 2 3 | 1.0 / 20 1.0 / 20 | Update scripts Q#32, Q#48, Q#49 & Q#71 Updated script T#22 & T#29 | December 28, 2018 |
| 2 | 1.0 / 21 | Updated script Q#100 | January 31, 2019 |
| 2 | 1.0 / 22 | Updated scripts Q#6, Q#7, Q#9 & Q#12 | May 1, 2019 |
| 2 | 1.0 / 23 | Added scripts Q#122, Q#123, and Q#124 | May 16, 2019 |
| 2 3 | 1.0 / 24 1.0 / 21 | Updated script Q#124 Updated script title T#6 | May 31, 2019 |
| 2 3 | 1.0 / 25 1.0 / 22 | Deleted Script Q#26 Added Scripts Q#106 & Q#107 Deleted Scripts T#39 & T#40 Added Scripts T#91 & T#92 | August 1, 2019 |
| 2 3 | 1.0 / 25 1.0 / 23 | Updated scripts Q#6, Q#7, Q#8, Q#9, Q#26, Q#58, Q#59, Q#62, Q#63, Q#66, Q#67, Q#75, Q#122, Q#123 & Q#124 and added scripts Q#26 & Q#125 Added script T#39 and updated scripts T#72, T#73 & T#85 | November 1, 2019 |
| 2 3 | 1.0 / 27 1.0 / 24 | Updated script Q#103, Q#117 & Q#122 Updated script T#88 | January 1, 2019 |
| 2 3 | 1.0 / 28 1.0 / 25 | Updated script Q#89 & Q#110 Updated Script T#63 | January 31, 2020 |
| 3 | 1.0 / 26 | Updated Script T#53 | May 29, 2020 |
| 2 3 | 1.0 / 29 1.0 / 27 | Added Scripts Q#126, Q#127 & Q#128 per NPRR930 Added Scripts T#101, T#102 & T#103 per NPRR930 | July 1, 2020 |
| 2 | 1.0 / 30 | Updated script Q#123 & Q#124 | October 1, 2020 |
| 2 3 | 1.0 / 31 1.0 / 28 | Updated Scripts Q#108 & Q#109 Updated Scripts T#97 & T#98 | December 31, 2020 |
| 2 | 1.0 / 32 | Updated Script Q#13 | April 1, 2021 |
| 2 3 | 1.0 / 33 1.0 / 29 | Deleted Q#110 & Q#111 Deleted T#63 & T#64 | July 31, 2021 |
| 2 3 | 1.0 / 34 1.0 / 30 | Updated Script Q#75 Updated Script T#85 | September 1, 2021 |
| 2 3 | 1.0 / 35 1.0 / 31 | Updated Script Q#126 Added Scripts Q#102, Q#109, Q#110,Q#129 & Q#130 Added Script T#40 | December 1, 2021 |
| 3 | 1.0 / 32 | Updated Script T#40 | December 17, 2021 |
| 2 3 | 1.0 / 36 1.0 / 33 | Updated Scripts Q#15, Q#62, Q#63, Q#64, Q#102 & Q#111 Updated Script T#12 All scripts in this manual have been reviewed | December 31, 2021 |
| 3 | 1.0 / 34 | Updated Scripts T#1, T#7, T#76, T#77, T#78 & deleted script T#40 | February 1, 2022 |
| 2 3 | 1.0 / 37 1.0 / 35 | Updated Script Q#8 Added Scrip Q#131 Added Script T#40 | May 26, 2022 |
| 2 3 | 1.0 / 38 1.0 / 36 | Updated Script Q#1, Q#2, Q#21, Q#110, & Q#111 Updated Script T#2 | July 29, 2022 |
| 2 3 | 1.0 / 39 1.0 / 36 | Added and updated scripts Q#5, Q#6, Q#7, Q#9, Q#64, Q#110, Q#111, Q132, & Q133 Added and updated scripts T#63, T#64, T#78, & T#87 All scripts in this manual have been reviewed | December 30, 2022 |
| 2 3 | 1.0 / 40 1.0 / 37 | Updated script Q#11 Updated script T#8 | February 1, 2023 |
| 2 3 | 1.0 / 41 1.0 / 38 | Q#62, Q#63, Q#64, Q#65, Q#66, Q#67, Q#68, Q#69, & Q#70 T#84 | March 31, 2023 |
| 2 | 1.0 / 42 | Updated script Q#126 | April 28, 2023 |
| 2 | 1.0 / 43 | Updated scripts Q#8, Q#26, Q#39, Q#42, Q#130, & Q#131 | June 9, 2023 |
| 2 3 | - / 44 1.0 / 39 | Updated scripts Q#5, Q#83 Updated scripts T#67, T#87 | June 30, 2023 |
| 2 3 | 1.0 / 45 1.0 / 40 | Updated script Q#10, Q#55 Updated Script T#34, T#39 | September 1, 2023 |
| 2 3 | 1.0 / 46 1.0 / 41 | Updated Script Q#7 Updated Script Q#55 Updated Script Q#56 Updated Scrpt T#34 Updated Script T#35 | October 4, 2023 |
| 2 3 | 1.0 / 47 1.0 / 42 | Updated Script Q#1, Q#2, Q#9, Q#11, Q#17, Removed Q# 3, Q# 4, Q# 12, Q# 103 Updated Script T#1, T#2, T#5, T#7, T#8, T#14, Removed T# 3, T# 4, T#6, T#88 | November 1, 2023 |