	ERCOT Operating Procedure Manual

	DC Tie Desk



Power Operations Bulletin # 2000
ERCOT has posted/revised the DC Tie manual.

The Various Changes are shown below.

A copy of the procedure can be found at:

http://www.ercot.com/mktrules/guides/procedures/index.html 
1.1
Purpose

The ERCOT DC Tie Desk Operator is responsible for energy transactions into and out of the ERCOT Control Area across the asynchronous DC Ties.  These transactions will be implemented in accordance with the North American Electric Reliability Corporation (NERC) Standard and the ERCOT Protocols and other Binding Documents.  The DC Tie Desk Operator will perform schedule confirmation with the applicable interconnected non-ERCOT Control Areas and will coordinate the approval process for NERC E-tags as both the ERCOT Control Area and on behalf of ERCOT TOs.  The DC Tie Desk Operator will also confirm interconnected non-ERCOT Control Area schedule profiles with the DC Tie Operators, who will control the DC Tie to the schedules agreed to by both the designated Reliability Coordinator for the interconnected non-ERCOT Control Area and ERCOT. 

The DC Tie Desk Operator performs its job function from the alternate control center and is also responsible for performing the loss of primary control center functionality procedure.

1.2
Scope

The instructions contained in this procedure are limited to those required for the DC Tie Desk Operator.  Instructions for other ERCOT control room positions are contained in separate procedures, one for each position.  This procedure does not imply that the duties contained herein are the only duties to be performed by this position.  The individual assigned to this position will be required to follow any other instructions and to perform any other duties as required or requested by appropriate ERCOT supervision.  

Although the steps within the procedures are numbered, the numbering is for indexing purposes and are not sequential in nature.  The System Operator will determine the sequence of steps, exclude steps, or take any additional actions required to ensure system security based on the information and situational awareness available during both normal and Emergency Conditions. 

2.1
System Operator Responsibility and Authority

Procedure Purpose: To ensure the System Operators know their roles, responsibility, and authority.

	Protocol Reference
	6.5.1.1
	6.5.1.2(3)
	6.5.2
	6.5.3(1)

	Guide Reference
	4.5.2(1)
	
	
	

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


ERCOT ISO as a Transmission Operator (TOP), the single Balancing Authority (BA), and only Reliability Coordinator (RC) registered within the ERCOT Interconnection shares all information between these roles simultaneously and acts concurrently as a single entity, satisfying coordination between the TOP, BA, and RC.

The System Operator (SO) shall, in accordance with NERC Reliability Standards and ERCOT Protocols, have clear decision-making authority to act to address the reliability of its Reliability Coordinator Area by direct actions or by issuing Dispatch Instructions / Operating Instructions during both normal and emergency conditions.  These actions shall be taken without delay and may include shedding of firm load without obtaining approval from higher-level personnel.  

The SO on duty is, in accordance with the ERCOT Protocols, Operating Guides, and NERC Reliability Standards, and acting as the Balancing Authority, Transmission Operator, and RC, shall request and receive information required to continually monitor the operating conditions which will assure security and reliability of the ERCOT system.

The SO issues Dispatch Instructions / Operating Instructions for the Real-Time operation of Transmission Facilities to a TO and to a QSE for the Real -Time operation of a Resource. 

The SO shall, on an ERCOT-wide basis, coordinate the ERCOT System Restoration (Black Start) Plan.  The SO shall implement the Black Start Plan and shall direct the reconnection efforts of the islands, established by restoration activities.  

The SO shall ensure compliance with the ERCOT Protocols and Operating Guides on identified transfer limits.  The SO receives all specified notifications for the MPs regarding energy transactions (Balanced Schedules), AGC control modes, routine adjustments, and equipment limitations and outages.

The SO performs security analyses on a Day Ahead and Real-Time basis and ensures all Forced Outages are entered into the Outage Scheduler. The SO shall obtain or arrange to provide Emergency Energy over the DC Tie(s) on behalf of ERCOT.

The SO shall issue appropriate OCN’s, AAN’s, Advisories, Watches, and Emergency Notices, and coordinate the reduction or cancellation of clearances, re-dispatch of generation, and request, order, or take other action(s) that the SO determines is necessary to maintain safe and reliable operating conditions on the ERCOT system in accordance with ERCOT Protocols, Operating Guides, and NERC Reliability Standards.  The SO will implement and terminate ERCOT Time Corrections and will determine the need for and implement the operation of a QSE on Constant Frequency Control for loss of ERCOT’s load frequency control system.


2.2
Communication

Procedure Purpose:  To ensure proper communication is used to reduce the possibility of miscommunication that could lead to action or inaction harmful to the reliability of the grid.

	Protocol Reference
	6.5.7.8
	
	
	

	Guide Reference
	3.1.3
	
	
	

	NERC Standard
	COM-002-4

R5, R7
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	Three-Part Communication

	NOTE
	· Operating Instructions and Dispatch Instructions are synonymous, and both require ‘three-part communication’.

· Refer to the Communications Protocols document for requirements.

	1
	When issuing Operating Instructions, use three-part communication:

· Issue the Operating Instruction

· Receive a correct repeat back

· Give an acknowledgement



	2
	Many scripts have been placed throughout the procedures as a reminder for the three-part communication.  However, a script cannot be provided for every scenario.  Effective three-part communication skills are mandatory.

	Hotline Call Communication

	1
	When making Hotline calls, ensure one TO/QSE repeats back the message.

IF:

· Time and circumstances allow;

THEN:

· Review the Consortium Hotline attendance report to verify all TOs/QSEs were in attendance

· Contact the TO/QSE using their OPX line or LD line to provide them with the message

· Inquire why they were not on the Hotline call

· Open a Help ticket if ERCOT’s Telecommunications department is needed to investigate.

	LOG
	Log all actions.

	Master QSE

	1
	Issue the VDI to the Master QSE of a Generation Resource that has been split to function as two or more Split Generation Resources as deemed necessary by ERCOT to effectuate actions for the total Generation Resource for instances when electronic Dispatch Instructions are not feasible.

	LOG
	Log all actions.


3.1
Approve E-Tags for the SPP DC Ties

Procedure Purpose: An E-Tag received in the OATI WebTrans regarding DC Tie flows into or out of ERCOT will be validated for accuracy.

	Protocol Reference
	4.4.4
	4.4.4.1
	4.4.4.2
	

	Guide Reference
	2.1(4)(e)
	
	
	

	NERC Standard
	INT-006-5 

R1, R1.1, R2, R4, R5, R5.1, R5.2, R5.3, R5.4, R5.5
	INT-009-3

R3
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	NOTE
	· AEP TOP located in Ohio is the DC Tie Operator (AEP DC TIE) for the DC_E (East tie) and DC_N (North tie)

· If either DC Tie is over-scheduled, contact the SPP Operator (SPP RTBM)  

	NOTE
	ERCOT has analyzed the Ramping capability of its Interconnection and is capable of Ramping at a greater rate than the maximum Ramp Rate schedule on any of the DC-Ties.

	NOTE


	Timing requirements can be found in NERC Standard INT-006-5 Attachment 1, the OATI software incorporates these timing requirements into the tool and actions cannot be taken outside the timing requirements. 

	1
	Automatic validation in WebTrans for each new or updated E-Tag will display ‘pass’ or ‘fail’.  The criteria used to automatically validate E-Tags is:

· ERCOT Market Information field must have QSE information

· TSP (Transmission Service Provider) must have ERCO specified in the Physical Path

· Tag Transaction Type must not be a Dynamic Schedule

IF:
· Either of the above validations fail; 

THEN:

· The E-Tag will be automatically denied. 

IF:
· You feel an E-Tag failed validation erroneously; 

THEN:

Go to the list of approved ERCOT Registered QSE’s at the following website:  https://wiki.ercot.com/pages/viewpageattachments.action?pageId=186982491
· Select “View” of the most recent post of “ERCOT QSE DC Tie Scheduling List” to view the ERCOT Registered QSE’s.,  

IF:
· The QSE name is on the list but was denied by WebTrans; 

THEN:

· Approve the E-Tag and inform the ERCOT Service desk and OATI Service desk if needed.

WebTrans Reference Display:

Validation > Tag Validation Monitor

	2
	When a New or Updated E-Tag is received in  WebTrans system:

Manually validate the following:

· Contact Information must be the QSE information and not ERCOT

· “ERCO” must not be listed as a PSE in Tag Information

· “ERCO” must not be listed as a PSE in the Physical Path

· Time Zone should be CS (central standard), CD (central daylight) or CP (central prevailing) during time change

· “Transaction type” must be “Energy or Normal”

· “ERCOT-E” or “ERCOT-N” must be selected for the correct POR (Point-of-Receipt) or POD (Point of Delivery) in the Physical Path 

·  “ERCOT-E” or “ERCOT-N” must be selected for the correct POR (Point-of-Receipt) or POD (Point of Delivery) on the ERCO TSP Row

· Do not allow same tie on the same row for the POR and POD, example: “ERCOTN to “ERCOTN”  

· There are no losses (MW) in the ERCOT Control Area.  This would be either to or from SPP as shown on the “Energy and Transmission Profiles” section, loss accounting may be blank, financial, or in-kind for ERCOT 

· E-Tags are permitted to be submitted within the COP time frame (up to seven days in advance), but a separate E-Tag for each day must be submitted and the Start Time and Stop Time MUST NOT cross the midnight hour

· If ERCOT is the sink BA, it is our responsibility to ensure FERC is cc’d on the E-Tag.

· If an E-Tag comes in with a Late status, deny the E-Tag (unless it is a curtailment).

IF:
· Any of the above information is incorrect; 

THEN:

· Deny the E-Tag, otherwise proceed below.

IF:
· An E-Tag is received for a time period when the DC Tie will be unavailable; 

THEN:

· Deny the E-Tag.

IF:
· One or more E-Tags are submitted that will cause the DC Tie to be overscheduled;

THEN:

· Contact SPP Tariff (SPP Tariff) approximately 30 minutes prior to the schedule start time to ensure they also see the over-schedule; they will take action to correct the over-schedule.  If the correction is not in place by 20 minutes before the over-schedule will occur, ERCOT will curtail the E-Tag to zero (0) based on the criteria of “Last in – First out” until the over schedule is alleviated.

IF:

· ERCOT curtails an E-Tag due to an over-schedule that was not corrected in time; 

THEN:

· Contact the source/sink control area to ensure the curtailment will be implemented as soon as possible.  

WebTrans Reference Displays: 

Scheduling>Interchange>BTF Checkout

	NOTE
	If verification needs to be made in the current day HRUC will be used unless it is a current hour E-Tag. If verification needs to be made for the next day DRUC will be used.

	NOTE
	Webtrans and MMS (MOI) utilize a different sign convention (positive and negative numbers) between the two systems.

OATI:

· (-) Imports 

· (+) Exports

MMS: 

· (+) Imports 

· (-) Exports

	COP           E-Tags
	REVIEW REFERENCE DISPLAY:

Market Operation>Reliability Unit Commitment> HRUC Displays> DSI Displays> DSI Data Processes> DSI DC Tie Schedule 

Market Operation>Reliability Unit Commitment> HRUC Displays> DSI Displays> External Input Data> MI COP

Verify that E-Tags are being imported into the MMS by going to the MI COP display and filter ‘ERCOT’ as the QSE name.


	East HVDC Tie Voltage Limits

	NOTE
	The East HVDC tie has voltage limits depending on the tie condition.

See Desktop Guide Transmission Desk 2.19

	1
	WHEN:

· Notified by AEP TOP that Reactors have been placed in/out of service on the East DC tie

THEN:

· Notify the Transmission desk operator

	LOG
	Log all actions.


3.2
Import and Export Limits for the CENACE DC Ties


Procedure Purpose: To confirm the import and export limits of the CENACE DC Ties to ensure Reliability of the ERCOT Interconnection.
	Protocol Reference
	4.4.4
	
	
	

	Guide Reference
	
	
	
	

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	NOTE
	Studies are required to set import/export limits between CENACE and ERCOT based on system conditions for commercial and emergency operations.  ERCOT will conduct studies in different time horizons including but not limited to seasonal (winter and summer peaks), Day-Ahead and real-time as needed. Operations Support group will calculate daily import/export transfer advisory limits for the Railroad and Laredo VFT. 

Revised Day-Ahead import and export advisory limits for the Railroad and Laredo VFT will be posted on the “Transmission Limits to Be Monitored” on the ERCOT Website at approximately 1700 in the Day-Ahead.

ERCOT Website>Data Products>GRID>Transmission>Generic Transmission Limits

If the limits are not posted, notify the Operations Support Engineer.

	Adjustment Period
	The Adjustment Period is for each Operating Hour, the time between 1800 in the Day-Ahead up to the start of the hour before that Operating Hour.  

	1
	Once the DC Tie revised limits for next day are received and prior to the start of the 1800 Day-Ahead Adjustment Period, check the sum of the E-Tags to ensure the export limits are not exceeded on any of the CENACE DC Tie.

IF:
· The sum of the E-Tags exceeds the revised import or export limit on any CENACE DC Tie, 

THEN:
· Anytime an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website.

· ERCOT will curtail the E-Tag based on the criteria of “Last in – First out” in real time as needed to maintain reliability.

	2
	IF:
· A new E-Tag is received in the Adjustment Period or Operating Hour which creates an exceedance of the physical capacity on any CENACE DC Tie, 

THEN:

· Deny the E-Tag.   


3.3
Approve E-Tags for the CENACE DC Ties

Procedure Purpose: An E-Tag received in the OATI WebTrans regarding DC Tie flows into or out of ERCOT will be validated for accuracy.

	Protocol Reference
	4.4.4
	4.4.4.1
	4.4.4.2(1)
	

	Guide Reference
	2.1(4)(e)
	
	
	

	NERC Standard
	INT-006-5 R1, R1.1, R2, R4, R5, R5.1, R5.2, R5.3, R5.4, R5.5
	INT-009-3

R3
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	NOTE
	· AEP TO is the DC Tie Operator (AEP TO) for the DC_L (Laredo VFT)

· ONCOR TO is the DC Tie Operator (ONCOR DC TIE) for the DC_R (Railroad DC Tie)

· All communications will be with the DC Tie Operator, and they will communicate with CENACE

	NOTE
	ERCOT will not allow an E-Tag to flow from CENACE to SPP or from SPP to CENACE.  This will require two separate E-Tags.  E-Tags are not calculated in the ACE equation, DC Tie schedules are seen by the EMMS as a generator (import) or load (export).

	NOTE
	· ERCOT has analyzed the Ramping capability of its Interconnection and is capable of Ramping at a greater rate than the maximum Ramp Rate schedule on any of the DC Ties

· The Railroad DC Tie has a 15 MW minimum flow requirement when E-Tags are scheduled

· Approximately one hour before start of schedule, notify the Transmission and Security Desk Operator with scheduled MW, Tie and direction of flow and duration.

	NOTE


	Timing requirements can be found in NERC Standard INT-006-5 Attachment 1, the OATI software incorporates these timing requirements into the tool and actions cannot be taken outside the timing requirements. 

	1 
	Automatic validation in WebTrans for each new or updated E-Tag will display ‘pass’ or ‘fail’.  The criteria ERCOT uses to validate E-Tags is:

· ERCOT Market Information fields must have QSE information

· TSP (Transmission Service Provider) must have ERCO, and CEN specified in the Physical Path

· E-Tag Transaction Type must not be a Dynamic Schedule

If either of the above validations fail, the E-Tag will be automatically denied. 

If you believe an E-Tag failed validation erroneously, the list of approved ERCOT Registered QSE’s can be found at the following website:

https://wiki.ercot.com/pages/viewpageattachments.action?pageId=186982491
Select “View” of the most recent post of “ERCOT QSE DC Tie Scheduling List” to view the ERCOT Registered QSE’s.  If the QSE name is on the list but was denied by WebTrans, approve the E-Tag, and inform the ERCOT Service desk and OATI Service desk if needed.

Another point of reference is to view the “QSE Contracts” in WebTrans.  Each QSE is associated with each DC Tie individually.

WebTrans Reference Display:

Tag Validation Summary

Data>QSE Contracts

	2
	When a New or Updated E-Tag is received in the WebTrans system:

Manually validate the following:

· PSE Contact Information must be the QSE information and not ERCOT

· “ERCO” must not be listed as a PSE in the E-Tag Information

· “ERCO” must not be listed as a PSE in the Physical Path

· Time Zone should be CS (central standard), CD (central daylight) or CP (central prevailing) during time change

· “Transaction type” must be “Energy or Normal”

· ERCO, AEPT and CEN must be included as the TSP (Transmission Service Provider) in the Physical Path and ERCO as a Scheduling Entity on the AEPT TSP row in the Physical Path for DC_L

· ERCO, ONC and CEN must be included as the TSP in the Physical Path and ERCO as a Scheduling Entity on the ONC TSP row in the Physical Path for the DC_R

· ERCOT and CFE/CEN must be listed as the Source and Sink under POR (Point-of-Receipt) and POD (Point of Delivery).

· There are no losses (MW) in the ERCOT Control Area.  This should be to or from AEPT or ONC as shown on the “Energy and Transmission Profiles” section, loss accounting may be blank, financial, or in-kind for ERCOT

· A separate E-Tag for each day must be submitted and the Start Time and Stop Time MUST NOT cross the midnight hour

· If the E-Tag comes in with a Late status, deny the E-Tag (unless it is a curtailment)

· The sum of E-Tags is submitted that will cause the DC Tie to exceed the physical capacity for the tie

· The Railroad DC Tie has a 15 MW minimum flow requirement.  Ensure the net of the E-Tags are not less than 15 MW

· Only ERCO and CEN are allowed as BA in the Physical Path, 

IF:
· Any of the above information is incorrect on the E-Tag; 

THEN:
· Deny the E-Tag.

IF:

· An E-Tag is denied due to another entity other than ERCO or CEN as BA in the Physical Path; 

THEN:

· Post the appropriate Preformatted DCTCN Notice to the ERCOT Website and

· Use the canned message,

· Send email to SysOpsMgrs to coordinate a Market Notice.

WebTrans Reference Displays:

Scheduling>Interchange>BTF Checkout

	3
	The Ramp Rate for DC_R will be 50 MW per minute due to reactive issues on the CENACE side.  DC_L has no Ramp limitations. 

· Ramp duration should be filled out on DC_R E-Tags only

· Deny any Railroad DC-Tie E-Tag that does not have a Ramp duration entered.

· DC_L Ramp duration can remain blank which is the default for a 10-minute Ramp.

	NOTE
	Webtrans and MMS (MOI) utilize a different sign convention (positive and negative numbers) between the two systems.

WebTrans:

· (-) Imports 

· (+) Exports

MMS: 

· (+) Imports 

· (-) Exports

	COP         E-Tags
	REVIEW REFERENCE DISPLAY:

Market Operation>Reliability Unit Commitment> HRUC Displays> DSI Displays> DSI Data Processes> DSI DC Tie Schedule 

Market Operation>Reliability Unit Commitment> HRUC Displays> DSI Displays> External Input Data> MI COP

· Verify that E-Tags are being imported into the MMS by going to the MI COP display and filter ‘ERCOT’ as the QSE name


3.5
Curtailment for the SPP DC Ties

Procedure Purpose: The process to curtail E-Tags for the SPP DC Ties for reliability purposes or DC Tie trips.
	Protocol Reference
	4.4.4
	
	
	

	Guide Reference
	
	
	
	

	NERC Standard
	INT-006-5

R3, R3.1
	IRO-006-5

R1
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	NOTE
	· Curtailments are customarily scheduled for one hour at a time.  Therefore, a curtailment that occurs several hours in a row may require that this procedure be repeated hourly

· If a System Operator in a non-ERCOT Control Area requests curtailment of a DC Tie Schedule due to an actual or anticipated emergency in its Control Area, ERCOT may curtail the DC Tie Schedule.  If the DC Tie Schedule is curtailed, ERCOT shall post a DC Tie Curtailment Notice to the ERCOT Website as soon as practicable
· SPP may make curtailments for Transmission Loading Relief (TLR) on their system.

	1
	IF:

· A curtailment alarm is received in WebTrans, AND complying with the request will NOT put ERCOT in an unreliable state (such as EEA or IROL exceedance);

THEN:

· Approve the E-Tag curtailment.

IF:

· A curtailment alarm is received in WebTrans, AND complying with the request WILL put ERCOT in an unreliable state (such as EEA or IROL exceedance);

THEN:

· Deny the E-Tag, stating the reason on the E-Tag, AND

· Call SPP RC and provide reliability reason for denying the request within 10 minutes. 

Webtrans Reference Displays: 

Tag Validation Summary

Tag Approval Monitor

	2
	IF:
· A QSE or SPP requests ERCOT to make an E-Tag change for them, either a curtailment or adjustment, 

THEN:

· Curtail the E-Tag to 0 (zero).

	NOTE
	E-Tag curtailments must be in an “Approved” or “Implemented” status to be calculated into the net scheduled interchange.

	3
	IF:

· The net schedule interchange value changes intra-hour, and

· Requested by the SPP Operator, 

THEN:

· Checkout the net scheduled interchange value for each SPP DC Tie affected by E-Tag curtailments.

WebTrans Reference Display:

Realtime Summary


3.6
Adjustment of CENACE DC Ties

Procedure Purpose: The process to approve or deny adjustments to E-Tags.
	Protocol Reference
	4.4.4
	
	
	

	Guide Reference
	2.8
	
	
	

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	Imports

	NOTE
	E-Tag adjustments must be in an “Approved” or “Implemented” status to be calculated into the net scheduled interchange.

	NOTE
	If a System Operator in a non-ERCOT Control Area requests curtailment of a DC Tie Schedule due to an actual or anticipated emergency in its Control Area, ERCOT may curtail the DC Tie Schedule.  If the DC Tie Schedule is curtailed, ERCOT shall post a DC Tie Curtailment Notice to the ERCOT Website as soon as practicable.

	1
	IF requested by either DC Tie Operator, AEP TO or ONCOR TO (ONCOR DC TIE), checkout the net scheduled interchange value for each CENACE DC Tie affected by E-Tag adjustments.

WebTrans Reference Display:

Realtime Summary

	Exports

	1
	IF:
· The adjustment causes the sum of the E-Tags to exceed the physical capacity export limit on any CENACE DC Tie, 

THEN:

· Deny the E-Tag adjustment.

	2
	Notify the Transmission and Security Desk Operator when E-Tag Net Export adjustments are greater than 100 MW during the Adjustment Period.

	NOTE
	E-Tag adjustments must be in an “Approved” or “Implemented” status to be calculated into the net scheduled interchange.

	NOTE
	If a System Operator in a non-ERCOT Control Area requests curtailment of a DC Tie Schedule due to an actual or anticipated emergency in its Control Area, ERCOT may curtail the DC Tie Schedule.  If the DC Tie Schedule is curtailed, ERCOT shall post a DC Tie Curtailment Notice to the ERCOT Website as soon as practicable.

	3
	IF requested by either DC Tie Operator (AEP TO or ONCOR DC Tie), checkout the net scheduled interchange value for each CENACE DC Tie affected by E-Tag adjustments.

WebTrans Reference Display:

Realtime Summary

	Post-Contingency Overloads

	NOTE
	DC Tie Load shall neither be curtailed during the Adjustment Period, nor for more than one hour at a time, except for the purpose of maintaining reliability.

	1
	IF:

· If requested by the Transmission and Security Desk Operator to curtail a South DC Tie to a specific MW amount to resolve a post-contingency overload.

THEN:

· Notify the appropriate DC Tie Operator (AEP TO or ONCOR DC TIE) to Ramp the Tie to the specific MW amount given from the Transmission and Security Desk Operator.

· Curtail the appropriate South DC Tie to the requested MW amount given from the Transmission and Security Desk Operator. 

· ERCOT will curtail the E-Tag based on the criteria of “Last in – First out”.

· Post the appropriate Preformatted DCTCN Notice to the ERCOT Website and

· Use the canned message,

	Cancel 

Message
	WHEN:

· A South DC Tie specific MW amount to resolve a post-contingency overload is no longer needed; 

THEN:

· Cancel DCTCN ERCOT Website posting message


3.9
Managing Daily Inadvertent Energy for the Laredo VFT

Procedure Purpose: To track any difference between the schedule net interchange and the actual net interchange.

	Protocol Reference
	4.4.4
	
	
	

	Guide Reference
	2.2.1(3)(c)(i)
	2.8(1) & (3)
	
	

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	Monitor
	Monitor the flows on the DC Ties to ensure the Ties are Ramping appropriately and match the net schedules.

	NOTE
	If the proper information is not listed when an E-Tag is created and submitted, the E-Tag may not be displayed correctly in all WebTrans displays.  Examples include but are not limited to a PSE not using the proper POR/POD in the Physical Path (LAR_VFT). 

	1
	Before the start of the hour, confirm the net-scheduled value for each DC Tie with the DC Tie Operator (AEP TO).  If there is a discrepancy, go over each E-Tag ID and MW amount for the affected DC Tie(s).

· If an E-Tag cannot be seen in WebTrans OR by the DC Tie Operator, work to resolve the problem OR ensure the E-Tag is curtailed to zero (0).

WebTrans Reference Displays:

Scheduling>QSE Summary

Scheduling>BTF Display

	2
	Check out the Actual SCADA data values for the previous hour with the DC Tie Operator (AEP TO) for each DC Tie.

· If ERCOT’s actual values are different from the DC Tie Operator’s actual values by less than 10 MWH, overwrite the actual value in the WebTrans system with a whole number.

· If ERCOT’s actual values are different from the DC Tie Operator’s actual values by 10 MWH or more and the reason is known (database load, loss of ICCP, etc.), overwrite the actual value in the WebTrans system. Log the appropriate information.

· If ERCOT’s actual values are different from the DC Tie Operator’s actual values by 10 MWH or more and the reason is unknown; ERCOT and the DC Tie Operator will initiate an investigation.  Log the appropriate information (time, date, total MW difference, DC Tie, name of the DC Tie Operator and any other relevant information).  Open an ERCOT Service Desk ticket for GMS Support to investigate problems with the meter information and notify Shift Supervisor. 

WebTrans Reference Displays:

Scheduling>Inadvertent>Tie Data

Scheduling>Interchange>ATF Checkout

	3
	After midnight, each day, check out the total scheduled and actual values for each DC Tie with the DC Tie Operator (AEP TO):  

IF:

· Both parties agree on the Scheduled In and Out values and on the Actual In and Out values;

THEN:

· Prepare report for the DC_L

· Select “[Tie name]” on the CA/UDI Interface

· Select “Schedule” on the “Checkout” dropdown 

· Check the bottom box on the totals row in Column SC

· Click the “Apply Checkout” button

· Proceed to Step 5.

WebTrans Reference Display:

Scheduling>Interchange>ATF Checkout

	4
	If there is a disagreement on the total scheduled and/or actual values with the DC Tie Operator, verify the values for each hour as necessary to find the discrepancy.  An effort should be made to work out all discrepancies. 

If the discrepancy is with the scheduled values:

· Check the box in Column SC for each hour that ERCOT is in agreement with the DC Tie Operator for the scheduled value.

· Leave the box unchecked in Column SC for each hour where there is a discrepancy with a scheduled value, and it cannot be worked out with the DC Tie Operator.  

· Click the “Apply Checkout” button.

· Log the appropriate information and send email to shiftsupv@ercot.com.

If the discrepancy is with an actual value, change the ERCOT value to match the DC Tie Operator’s value.

WebTrans Reference Displays:

Scheduling>Interchange>ATF Checkout

Scheduling>Inadvertent>Tie Data

	5
	Create an Excel file (Export As Shown) for the  DC_L DC Tie and email to the following (see Desktop Guide DC Tie Desk Section 2.9):

· CFE DC Tie Inadvertent


3.10
Managing Daily Inadvertent Energy for the Railroad Tie

Procedure Purpose: To track any difference between the schedule net interchange and the actual net interchange.

	Protocol Reference
	4.4.4
	
	
	

	Guide Reference
	2.2.1(3)(c)(i)
	2.8(1) & (3)
	
	

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	Monitor
	Monitor the flows on the DC Ties to ensure the Ties are Ramping appropriately and match the net schedules.

	NOTE
	If the proper information is not listed when an E-Tag is created and submitted, the E-Tag may not be displayed correctly in all WebTrans displays.  Examples include but are not limited to a PSE not using the proper POR/POD in the Physical Path (ROAD_DC). 

	1
	Before the start of the hour, confirm the net-scheduled value for the DC Tie with the DC Tie Operator (ONCOR DC TIE).  If there is a discrepancy, go over each E-Tag ID and MW amount.

· If an E-Tag cannot be seen in WebTrans OR by the DC Tie Operator, work to resolve the problem OR ensure the E-Tag is curtailed to zero (0).

· Anytime an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website. 

· Use the canned message

WebTrans Reference Displays:

Scheduling>QSE Summary

Scheduling>BTF Display

	2
	Check out the Actual SCADA data values for the previous hour with the DC Tie Operator, ONCOR DC TIE for the Railroad DC Tie.

· If ERCOT’s actual values are different from the DC Tie Operator’s actual values by less than 10 MWH, overwrite the actual value in the WebTrans system with a whole number.

· If ERCOT’s actual values are different from the DC Tie Operator’s actual values by 10 MWH or more and the reason is known (database load, loss of ICCP, etc.), overwrite the actual value in the WebTrans system. Log the appropriate information.

· If ERCOT’s actual values are different from the DC Tie Operator’s actual values by 10 MWH or more and the reason is unknown; ERCOT and the DC Tie Operator will initiate an investigation.  Log the appropriate information (time, date, total MW difference, DC Tie, name of the DC Tie Operator and any other relevant information).  Open an ERCOT Service Desk ticket for GMS Support to investigate problems with the meter information and notify Shift Supervisor. 

WebTrans Reference Displays:

Scheduling>Inadvertent>Tie Data

Scheduling>Interchange>ATF Checkout

	3
	After midnight, each day, check out the total scheduled and actual values for the Railroad DC Tie with the DC Tie Operator (ONCOR DC TIE):  

IF:

· Both parties agree on the Scheduled In and Out values and on the Actual In and Out values;

THEN:

· Prepare report for the DC_R 

· Select “[Tie name]” on the CA/UDI Interface

· Select “Schedule” on the “Checkout” dropdown 

· Check the bottom box on the totals row in Column SC

· Click the “Apply Checkout” button

· Proceed to Step 5.

WebTrans Reference Display:

Scheduling>Interchange>ATF Checkout

	4
	If there is a disagreement on the total scheduled and/or actual values with the DC Tie Operator, verify the values for each hour as necessary to find the discrepancy.  An effort should be made to work out all discrepancies. 

If the discrepancy is with the scheduled values:

· Check the box in Column SC for each hour that ERCOT is in agreement with the DC Tie Operator for the scheduled value.

· Leave the box unchecked in Column SC for each hour where there is a discrepancy with a scheduled value, and it cannot be worked out with the DC Tie Operator.  

· Click the “Apply Checkout” button.

· Log the appropriate information and send email to shiftsupv@ercot.com.

If the discrepancy is with an actual value, change the ERCOT value to match the DC Tie Operator’s value.

WebTrans Reference Displays:

Scheduling>Interchange>ATF Checkout

Scheduling>Inadvertent>Tie Data

	5
	Create an Excel file (Export As Shown) for the DC-R Tie and email to the following (see Desktop Guide DC Tie Desk Section 2.9):

· CENACE DC Tie Inadvertent


3.11
Managing Inadvertent Energy Payback

Procedure Purpose: To provide communication contacts and information prior to starting and ending inadvertent payback. 

	Protocol Reference
	
	
	
	

	Guide Reference
	2.8 (3)
	
	
	

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	NOTE
	Payback will only be planned when there is room on the DC Tie(s).  Energy schedules of market participants will supersede any inadvertent payback.

	NOTE
	When inadvertent needs to be paid back in either direction, the ERCOT Operator will receive the payback plan information from the Director Control Room Operations and/or Designee.

	CENACE

DC Ties
	· Prior to the start of the payback plan, call the appropriate DC Tie Operator and confirm the start time, hourly MW amount, and direction of the payback.  

· If payback is planned across the DC_L, verify the transmission path (230kV or 138kV)

· Prior to the Payback plan stopping, call the appropriate entity and confirm the agreed upon stop time.

	SPP

DC Ties
	· Prior to the start of the payback plan, call the SPP Operator (SPP Tariff) and confirm the start time, hourly MW amount and direction of the payback


3.12
Loss of E-Tagging

Procedure Purpose: Contingency plan for loss of E-Tagging capability.
	Protocol Reference
	
	
	
	

	Guide Reference
	
	
	
	

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	Loss of ERCOT E-Tagging

	1
	If the WebTrans system goes down for an unknown reason (i.e., maintenance), contact the ERCOT Operator at the primary control center to check on their connectivity.  If their system is also down, contact the ERCOT Service desk (ERCOT HELPDESK) to see if the issue is on our end.  If issue is not on the ERCOT end, contact the OATI Service desk (OATI HELPDESK) to see if the issue is on their end.

	2
	Determine the estimated down time, if longer than 30 minutes, notify SPP (SPP Tariff), AEP Corpus (AEP TO), and ONCOR (ONCOR DC TIE).

	3
	Post message on the ERCOT Website:

Typical ERCOT Website Posting:
“The [ERCOT/OATI] E-Tagging system is down, and no new schedules will be accepted across the DC Ties at this time”.

	4
	No new E-Tags will be accepted, only curtailments:

· Coordinate with SPP (SPP Tariff) and the appropriate DC Tie Operator

· Notify the Real-Time Desk Operator to determine if SCED needs to be manually executed for curtailments 

	NOTE
	Before-the-fact and after-the-fact check outs will not be available

	5
	Once the WebTrans system is available, confirm SPP (SPP Tariff), AEP Corpus (AEP TO), and ONCOR (ONCOR DC TIE) have availability.  If available for all parties:

· Update the ERCOT Website posting by sending a “Notice Cancellation”

· If curtailments were made while the WebTrans system was down, ensure E-Tags are updated

	Loss of DC Tie Operator E-Tagging

	1

If the E-Tagging system is down for SPP, AEP Corpus or ONCOR DC Tie and not ERCOT, no new schedule can be accepted.

2

Post message on the ERCOT Website:

Typical ERCOT Website Posting:
“The E-Tagging system is down for the [state DC Tie Operator] and no new schedules will be accepted across the [state DC Tie(s)] at this time”.

3

Once E-Tagging system is operational, cancel message.




4.1
Implement EEA Levels 

Procedure Purpose: Steps to perform when ERCOT declares Energy Emergency Alert (EEA). 
	Protocol Reference
	4.4.4
	6.5.9.4.2
	6.5.9.4.3
	

	Guide Reference
	2.1(3)(f)
	4.5.3(2)
	4.5.3.3
	

	NERC Standard


	EOP-011-2

R2, R2.1, R2.2, R2.2.3, R2.2.3.1, R2.2.7
	INT-006-5

R1, R2, R3
	IRO-001-4

R1
	TOP-001-6

R2


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	Turret

Phone
	· The DC Tie Operator for the Laredo DC Tie can be reached using the Turret phone button labeled (AEP TO), page 4 

· The DC Tie Operator for the Railroad DC Tie can be reached using the Turret phone button labeled (ONCOR DC TIE), page 14

· AEP un-regulated QSE is responsible for assets associated with unregulated retail operations (SQ2, SQ3, and SQ4 sub-QSEs).  The electronic Dispatch Instruction for emergency energy to and from CENACE will be given to this QSE.  They can be reached using the Turret phone button labeled (AEP UNREG QSE), page 4  

	EEA
	When notified by Hotline that ERCOT has implemented EEA 1, proceed with this procedure.  See Desktop Guide DC Tie Desk Section 2.8 for flow diagram.

	DC Ties with Centro Nacional de Control de Energia (CENACE)

	1
	This applies to the Laredo VFT:

IF:

· There are export schedules, OR

· Export inadvertent payback flowing;

THEN:

· Stop the export inadvertent payback

· Curtail the export schedules (see Desktop Guide DC Tie Desk Section 2.3 or 2.4)

· Anytime an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website. 

· Use the canned message

CONTACT:

· The DC Tie Operator (AEP TO)

Typical script (use what is applicable):
This is ERCOT Operator [first and last name].  Due to ERCOTs EEA status:

· We have curtailed E-Tag(s) [tag number(s)] on the [DC Tie name] and request Ramping the Tie(s) accordingly

· We request the inadvertent payback on [DC Tie] be Ramped to zero

· We request you inquire about the availability of emergency energy from CENACE. 

Please repeat this back to me.  That is correct.  Thank you.”  

	2
	This applies to the Railroad DC Tie:

IF:

· There are export schedules, OR

· Export inadvertent payback flowing;

THEN:

· Stop the export inadvertent payback

· Curtail the export schedules (see Desktop Guide DC Tie Desk Section 2.3 or 2.4)

· Anytime an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website. 

· Use the canned message

CONTACT:

· The DC Tie Operator (ONCOR DC TIE)

Typical script (use what is applicable):
This is ERCOT Operator [first and last name].  Due to ERCOTs EEA status:

· We have curtailed E-Tag(s) [tag number(s)] and request Ramping the Tie accordingly
· We request the inadvertent payback be Ramped to zero
· We request you inquire about the availability of emergency energy from CENACE. 
Please repeat this back to me.  That is correct.  Thank you.”  

	Electronic

Dispatch

Instruction
	IF:

· AEP TO and/or ONCOR DC Tie notifies ERCOT of available emergency energy;

THEN:

· Accept all available emergency energy

· Notify the Shift Supervisor

· Issue the electronic Dispatch Instruction as time permits

Verbal Dispatch Instructions [Emergency] MOI display

AEP un-regulated QSE (AEP UNREG QSE):

· Choose DC Tie tab

· Choose QAEPS2 for Participant Name

· Choose DC Tie Name

· Enter Up-To MW value

· Choose PROVIDE EMERGENCY POWER FROM DC TIE – Import for Instruction Type

· State “Emergency Energy to ERCOT on [DC Tie(s)] in Other Information, Completion Time can be left blank, if unknown and inserted when completion time is known.(back office will take care of the rest)

· Click “Send” (do not “Commit to Database”)

Notify AEP un-regulated QSE (AEP UNREG QSE) that the electronic Dispatch Instruction has been issued. 

When issuing a VDI or confirmation, ensure the use of three-part communication.

	3
	This applies to the Laredo DC Tie:

WHEN:

· EEA has ended;

THEN:

· Notify the DC Tie Operator (AEP TO)

Typical script (use what is applicable):
 “This is ERCOT Operator [first and last name].  At [time], ERCOT has ended EEA 1.  Thank you.”

CONTACT:

· The DC Tie Operator (AEP TO) for the following as appropriate:

· End emergency energy

· Verify Completion Time from the Verbal Dispatch Instructions [Emergency] MOI display has a Completion Time entered for the appropriate DC Tie.

· Reload any E-Tag (see Desktop Guide DC Tie Section 2.5)

· Cancel DCTCN ERCOT Website posting message

· Re-start inadvertent payback

	4
	This applies to the Railroad DC Tie:

WHEN:

· EEA has ended;

THEN:

· Notify the DC Tie Operator (ONCOR DC TIE)
Typical script (use what is applicable):
 “This is ERCOT Operator [first and last name].  At [time], ERCOT has ended EEA 1.  Thank you.”

CONTACT:

· The DC Tie Operator (ONCOR DC TIE) for the following as appropriate:

· End emergency energy

· Verify Completion Time from the Verbal Dispatch Instructions [Emergency] MOI display has a Completion Time entered for the DC-R Tie.

· Reload any E-Tag (see Desktop Guide DC Tie Section 2.5)

· Cancel DCTCN ERCOT Website posting message

· Re-start inadvertent payback

	LOG
	Log all actions taken.

	DC Ties with Southwest Power Pool (SPP)

	Turret

Phone
	· SPP Reliability Coordinator can be reached using the Turret phone button labeled (SPP RC), page 2

· AEP regulated QSE is responsible for assets associated with regulated retail operations (Main and SQ1sub-QSE).  The electronic Dispatch Instruction for emergency energy from SPP will be given to this QSE.  They can be reached using the Turret phone button labeled (AEP REG QSE), page 4

	NOTE
	For any EEA level, only hourly non-firm transmission service is available for the North and East DC Ties. Keep AEP informed so they can make the proper submittals hour to hour.

	NOTE
	If an E-Tag cannot be timely submitted during an Emergency Condition, and energy is available.  The DC Tie Operator may start the flow after approval from the RC’s.  An E-Tag must be submitted after the schedule has physically started for E-Tag approval.

	1
	NOTIFY:

· SPP Reliability Coordinator (SPP RC) of ERCOT’s EEA 1 status:

Typical Script to SPP Reliability Coordinator (SPP RC):
“This is ERCOT Operator [first and last name].  At [time], ERCOT declared EEA 1”.

 

	2
	IF:

· There are export E-Tags across the DC Ties (does not apply to DC_N schedules which are identified by “OKLA” in the Physical Path on the tag), OR
· Export inadvertent payback flowing, OR

· DC_N deadband is active and exporting;

CONTACT:

· The SPP Tariff Operator (SPP Tariff)

Typical script (use what is applicable):
This is ERCOT Operator [first and last name].  ERCOT is in EEA [level]:

· We are curtailing E-Tag(s) [tag number(s)] and request that you Ramp the Tie accordingly
· We request the inadvertent payback be Ramped to zero
· If applicable, request the deadband on the North Tie be turned into ERCOT. 
Please repeat this back to me.  That is correct.  Thank you.”  
IF:

· SPP provides a reliability reason for NOT approving the curtailing of the E-Tag(s);

THEN:

· Do not proceed with curtailments

· Notify the Shift Supervisor

· Make a log entry

IF:

· SPP approves;

THEN:

· Curtail the export schedules (usually done one hour at a time) (see Desktop Guide DC Tie Desk Section 2.3 or 2.4)

· During an emergency condition if an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website. 

· Use the canned message

	3
	IF:

· The SPP DC Ties are not at maximum import into ERCOT;

THEN:

· Notify the AEP regulated QSE (AEP REG QSE) to request emergency energy.

Typical Script to AEP (AEP REG QSE):
“This is ERCOT Operator [first and last name].  At [time], ERCOT declared EEA [state level].  Can AEP assist ERCOT with transmission service and energy on either available DC Tie?” 

NOTE: AEP will have to request the transmission service and energy first before they will know if it is available.  SPP will approve if both services are available.

IF:
· Not approved;

THEN:

· Do NOT proceed and make log entry

· Notify Shift Supervisor

	Electronic

Dispatch

Instruction
	IF: 

· AEP notifies ERCOT of available emergency energy

THEN:

· Accept all available emergency energy

· Notify the Shift Supervisor

· Approve the E-Tag 

· Issue electronic Dispatch Instruction as time permits

Verbal Dispatch Instructions [Emergency] MOI display

AEP REG QSE:

· Choose DC Tie tab

· Choose QAEPSC for Participant Name 

· Choose DC Tie Name

· Enter Up-To MW value

· Choose PROVIDE EMERGENCY POWER FROM DC TIE - Import for Instruction Type

· State “Emergency into ERCOT” and which DC Tie(s) in Other Information

· Completion Time can be left blank, if unknown and inserted when completion time is known (back office will take care of the rest)

· Click “Send” (do not “Commit to Database”)

Notify AEP regulated QSE (AEP REG QSE) that the electronic Dispatch Instruction has been issued.

When issuing a VDI or confirmation, ensure the use of three-part communication.

	4
	WHEN:

· EEA status changes;

THEN:

· Notify SPP Reliability Coordinator (SPP RC)

Typical Script to SPP Reliability Coordinator (SPP RC):
“This is ERCOT Operator [first and last name].  At [time], ERCOT moved from EEA 1 to EEA 2.  Thank you”.

If moved to EEA 2 or 3 continue process above but with new levels.

	5
	WHEN:

· EEA has ended;

THEN:

· Notify SPP Reliability Coordinator (SPP RC)

Typical Script to SPP Reliability Coordinator (SPP RC):
“This is ERCOT Operator [first and last name].  At [time], ERCOT has ended EEA 1.  Thank you.”

CONTACT:

· The SPP Tariff Operator (SPP Tariff) for the following as appropriate:

· End emergency energy

· Verify Completion Time from the Verbal Dispatch Instructions [Emergency] MOI display has a Completion Time entered for the appropriate DC Tie

· Reload any E-Tag (see Desktop Guide DC Tie Section 2.5)

· Cancel DCTCN ERCOT Website posting message

· Re-start inadvertent payback

· If applicable, have deadband turned back into SPP

	LOG
	Log all actions taken.


4.2
Emergency Energy to CENACE

Procedure Purpose: Provide Emergency Energy to CENACE across the DC Ties.
	Protocol Reference
	4.4.4
	
	
	

	Guide Reference
	
	
	
	

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	NOTE
	If CENACE requests emergency energy, no emergency notification needs to be declared by ERCOT.  See Desktop Guide DC Tie Desk Section 2.8 for flow diagram.

	1
	If notified by the Transmission and Security Desk Operator or a DC Tie Operator(AEP TO or ONCOR DC TIE) of a request from CENACE for Emergency Energy across a specific DC Tie:

IF:

· There are import schedules, OR

· Import inadvertent payback flowing;

THEN:

· Curtail the import schedules (see Desktop Guide DC Tie Section 2.3 or 2.4) 

· Anytime an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website.

· Use the canned message
CONTACT:

· The DC Tie Operator (AEP TO or ONCOR DC Tie)

Typical script to appropriate DC Tie Operator (use what is applicable):
This is ERCOT Operator [first and last name].  Due to CENACEs emergency status:

· We have curtailed E-Tag(s) [tag number(s)] on the [DC tie name]

· We request the inadvertent payback on [DC Tie] be Ramped to zero

· We [are able/not able] to send emergency energy on [DC Tie

· If sending emergency energy, request the DC Tie be Ramped to the agreed amount

Please repeat this back to me.  That is correct.  Thank you.”  

	Electronic

Dispatch

Instruction
	Verbal Dispatch Instructions [Emergency] MOI display
Issue to AEP un-regulated QSE (AEP UNREG QSE):

· Choose DC Tie tab

· Choose QAEPS2 for Participant Name

· Choose DC Tie Name

· Enter Up-To MW value 

· Choose PROVIDE EMERGENCY POWER FROM DC TIE - Export for Instruction Type

· State “Emergency Energy to CENACE” and which DC Ties in “Other Information”

· Completion Time can be left blank, if unknown and inserted when completion time is known

· Click “Send” (do not “Commit to Database”)

Notify AEP un-regulated QSE (AEP UNREG QSE) that electronic Dispatch Instruction has been issued.

When issuing a VDI or confirmation, ensure the use of three-part communication. 

	2
	WHEN: 

· Notified that CENACEs emergency has ended; 

THEN:

· Perform/verify the following, if appropriate:

· End emergency energy

· Verify Completion Time from the Verbal Dispatch Instructions [Emergency] MOI display has a Completion Time entered for the appropriate DC Tie.

· Re-load E-Tag(s) (see Desktop Guide DC Tie Section 2.5)

· Cancel DCTCN ERCOT Website posting message

· Restart import inadvertent payback

	Log
	Log all actions taken.


4.3
Emergency Condition in the Areas of the CENACE DC Ties

Procedure Purpose: The process to curtail E-Tags for the CENACE Ties and request emergency energy, if needed.
	Protocol Reference
	4.4.4
	6.5.9.3.4
	
	

	Guide Reference
	2.8(2)
	
	
	

	NERC Standard
	EOP-011-2 R1, R1.1, R1.2, R1.2.4
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	Turret Phone
	· The DC Tie Operator for the Laredo DC Tie can be reached using the Turret phone button labeled (AEP TO), page 4 

· The DC Tie Operator for the Railroad DC Tie can be reached using the Turret Phone button labeled (ONCOR DC TIE), page 14

· AEP un-regulated is the QSE receiving the VDI for emergency energy is issued to and from CENACE.  They can be reached using the Turret phone button labeled (AEP UNREG QSE), page 4  

	NOTE
	DC Tie Load shall neither be curtailed during the Adjustment Period, nor for more than one hour at a time, except for the purpose of maintaining reliability.

	NOTE
	When ERCOT curtails an E-Tag whether to zero (0) or a specific MW amount, only CENACE needs to approve the change.  Anytime an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website.  See Desktop Guide DC Tie Desk Section 2.8 for flow diagram.  

	Post-Contingency Overloads on the South DC Ties

	1
	When post contingency overloads on the south DC Tie(s) (DC_L and/or DC_R), it may be necessary to curtail export E-Tags using the Last-in-First-Out (LIFO).  

IF:
· There is a need to curtail (which includes reductions) exports on a specific DC Tie;

THEN:

· Coordinate with the Transmission and Security Desk Operator

· Verify the MW amount the E-Tag needs to be curtailed 

· Curtail the appropriate E-Tag to the MW amount needed (see Desktop Guide DC Tie Section 2.3 or 2.4)

· Post the appropriate Preformatted DCTCN Notice to the ERCOT Website 

· Use the canned message

· Notify the appropriate DC Tie Operator with curtailment information and instruct them to Ramp the DC Tie and to notify CENACE

· Notify PSE with curtailment information and reason. 

Typical script to appropriate DC Tie Operator:
This is ERCOT Operator [first and last name].  ERCOT has curtailed E-Tag(s) [tag number(s)] on the [DC Tie name] to [state MW amount] starting at [xx:xx] due to a Transmission Watch issued for the South DC Tie(s) post contingency loss of [constraint name].  Please Ramp the Tie accordingly and notify CENACE. Please repeat this back to me.  That is correct.  Thank you.”  

	Transmission Watch Issued for the Rio Grande Valley Import

	NOTE
	When ERCOT curtails an E-Tag whether to zero (0) or a specific MW amount, only CENACE needs to approve the change.  Anytime an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website. 

	1
	When ERCOT has issued a Transmission Watch for the Rio Grande Valley Import, it may be necessary to curtail export E-Tags using the Last-in-First-Out (LIFO).  

IF:
There is a need to curtail (which includes reductions) exports on a specific DC Tie

THEN:

· Coordinate with the Transmission and Security Desk Operator

· Verify the MW amount the E-Tag needs to be curtailed 

· Curtail the appropriate E-Tag to the MW amount needed (see Desktop Guide DC Tie Section 2.3 or 2.4)

· Post the appropriate Preformatted DCTCN Notice to the ERCOT Website 

· Use the canned message

· Notify the appropriate DC Tie Operator with curtailment information and instruct them to Ramp the DC Tie and to notify CENACE

· Notify PSE with curtailment information and reason. 

Typical script to appropriate DC Tie Operator:
This is ERCOT Operator [first and last name].  ERCOT has curtailed E-Tag(s) [tag number(s)] on the [DC Tie name] to [state MW amount] starting at [xx:xx] due to a Transmission Watch issued for the Rio Grande Valley Import.  Please Ramp the Tie accordingly and notify CENACE. Please repeat this back to me.  That is correct.  Thank you.”  

	Emergency Condition within ERCOT

	NOTE
	When ERCOT curtails an E-Tag whether to zero (0) or a specific MW amount, only CENACE needs to approve the change.  Anytime an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website. 

	1
	When ERCOT is in an emergency, it may be necessary to curtail export E-Tags using the Last-in-First-Out (LIFO).  

IF:
· If there is a need to curtail (which includes reductions) exports on a specific DC Tie;

THEN:

· Coordinate with the Transmission and Security Desk Operator

· Verify the MW amount the E-Tag needs to be curtailed 

· Curtail the appropriate E-Tag to the MW amount needed (see Desktop Guide DC Tie Section 2.3 or 2.4)

· Post the appropriate Preformatted DCTCN Notice to the ERCOT Website 

· Use the canned message

· Notify the appropriate DC Tie Operator with curtailment information and instruct them to Ramp the DC Tie and to notify CENACE

· Notify PSE with curtailment information and reason. 

Typical script to appropriate DC Tie Operator:
This is ERCOT Operator [first and last name].  ERCOT has curtailed E-Tag(s) [tag number(s)] on the [DC Tie name] to [state MW amount] starting at [xx:xx] due to an Emergency condition within ERCOT.  Please Ramp the Tie accordingly and notify CENACE. Please repeat this back to me.  That is correct.  Thank you.”  

	2
	Coordinate with the Transmission and Security Desk Operator when the E-Tags can be re-loaded:

· Notify PSE with reload information

· Re-load curtailed E-Tag(s) (see Desktop Guide DC Tie Section 2.5)

· Cancel DCTCN ERCOT Website posting message

· Notify the appropriate DC Tie Operator to approve the re-loaded E-Tag and instruct them to notify CENACE.

Typical script to appropriate DC Tie Operator:
This is ERCOT Operator [first and last name].  At [xx:xx], the Emergency conditions within ERCOT has ended and the E-Tag(s) [tag number(s)] have been re-loaded for [xx:xx].  Please notify CENACE to approve re-load.  Please repeat this back to me.  That is correct.  Thank you.”  

	3
	IF:

· The re-loaded E-Tag(s) go passive deny because DC Tie Operator or CENACE did not approve the E-Tag(s) in the allotted time;

THEN:

· Re-load the E-Tag(s) again, AND

· Notify appropriate DC Tie Operator.
If reload fails, the second time stop trying and keep E-Tag curtailed.

	4
	IF:

· There is need for emergency energy across a specific DC Tie;

THEN:

· Coordinate with the Transmission and Security Desk Operator 
· Request the appropriate DC Tie Operator to inquire about the availability of Emergency Energy on [name DC Tie(s)]

Typical Script to appropriate DC Tie Operator:
“This is ERCOT Operator [first and last name] At [xx:xx] ERCOT declared an Emergency, we are requesting you to contact CENACE to inquire about the availability of Emergency Energy into ERCOT on [name DC Tie(s)]

IF:

· Emergency Energy is available;
THEN:

· Request the appropriate DC Tie Operator to Ramp the appropriate DC Tie(s) with the amount of emergency energy that CENACE can send,
· Issue electronic Dispatch Instruction
IF:

· Emergency energy is not available;

THEN:

· Notify the Transmission and Security Desk Operator

	5
	Verbal Dispatch Instructions [Emergency] MOI display
Issue VDI to AEP un-regulated QSE (AEP UNREG QSE) for the emergency energy:
· Choose DC Tie tab
· Choose QAEPS2 for Participant Name
· Choose DC Tie Name

· Enter Up-To MW value
· Choose PROVIDE EMERGENCY POWER FROM DC TIE - Import for Instruction Type
· State “Emergency Energy to ERCOT on [DC Tie(s)] in Other Information
· Completion time can be left blank, if unknown and inserted when completion time is known (back office will take care of rest)
· Click “Send” (do not “Commit to Database”)

Notify AEP un-regulated QSE (AEP UNREG QSE) that the electronic Dispatch Instruction has been issued.

When issuing a VDI or confirmation, ensure the use of three-part communication.

	LOG
	Log all actions taken.

	Emergency Condition within CENACE

	NOTE
	On the CENACE side of the Railroad DC Tie, there is an automatic runback scheme that runs back the DC Tie under CENACE contingency conditions.  When this occurs, and notification is made by the DC Tie Operator; follow the procedure below.

	NOTE
	When ERCOT curtails an E-Tag whether to zero (0) or a specific MW amount, only CENACE needs to approve the change.  Anytime an approved E-Tag is curtailed or adjusted by ERCOT a DC Tie Curtailment Notice (DCTCN) must be posted on the ERCOT Website.

	1
	Due to reliability issues within CENACE it may be necessary to curtail import E-Tag(s) for them.  

IF:
· CENACE, communicated by one of the DC Tie Operators needs to curtail an import on a specific CENACE DC Ties;

THEN:

· Coordinate with the Transmission and Security Desk Operator

· Verify the MW amount and time of the curtailment

· Curtail the appropriate E-Tag to the MW amount needed (see Desktop Guide DC Tie Section 2.3 or 2.4),

· Post the appropriate Preformatted DCTCN Notice to the ERCOT Website 

· Use the canned message

· Contact the appropriate DC Tie Operator when curtailment is complete and instruct them to Ramp the DC Tie and make notification to CENACE 
· Notify PSE with curtailment information and reason.
Typical script to appropriate DC Tie Operator:
This is ERCOT Operator [first and last name].  At [xx:xx] ERCOT has curtailed E-Tag [tag number] on the [DC Tie name] to [state MW amount] due to reliability issues within CENACE.  Please Ramp the Tie accordingly and notify CENACE. Please repeat this back to me.  That is correct.  Thank you.”  

	2
	Once notified that CENACEs reliability issue has been resolved and E-Tags can be re-loaded:

· Coordinate with the Transmission and Security Desk Operator

· Notify PSE with reload information

· Re-load curtailed E-Tag(s) (see Desktop Guide DC Tie Desk Section 2.5),

· Cancel DCTCN ERCOT Website posting message

· Notify the appropriate DC Tie Operator to approve the re-loaded E-Tag and to notify CENACE.

	3
	IF:

· The re-loaded E-Tag goes passive deny because a DC Tie Operator or CENACE did not approve the E-Tag in the allotted time;

THEN:

· Re-load the E-Tag again, AND

· Notify the appropriate DC Tie Operator.

If reload fails, the second time stop trying and keep E-Tag curtailed.

	4
	IF:

· There is a need for emergency energy;

THEN:

· Proceed and follow step 1 in section 4.2, Emergency Energy to CENACE.

	LOG
	Log all actions taken.


4.4
Loss of Primary Control Center Functionality
Procedure Purpose: Provide instructions for responding to conditions that cause the primary control center to become inoperable or uninhabitable while continuing to meet its obligations with regard to the reliable operations of the ERCOT system and ensuring the safety of control center personnel.

	Protocol Reference
	
	
	
	

	Guide Reference
	
	
	
	

	NERC Standard
	EOP-008-2

R1, R1.2, R1.2.1, R1.4 R1.6, R1.6.1, R1.6.2, 1.6.3, R4
	TOP-001-6

R9
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	Contact Security
	IF:

· Unable to reach any Operator at the PCC (also trying Alternative Interpersonal Communication capability);

THEN:

· Check the control center video camera to determine the status of the PCC and personnel,

· Contact Security at the ACC

· Notify them of the situation AND
· Have them attempt to make contact with Security at PCC.

· Have them contact you with information acquired OR
IF:

· Notified by the PCC that they are evacuating and/or transferring sites OR
· control center video and Security confirms evacuation;

THEN:

· Continue with procedure.  

	3-Part


	When issuing an Operating Instruction, follow  three-part communication:

· Issue the Operating Instruction

· Receive a correct repeat back

· Give an acknowledgement

	Systems are Functional

	Call in Additional Operators
	Coordinate with the Transmission and Security Desk Operator and activate the NXT scenario to call in additional shift personnel, which includes a Shift Supervisor. 

 SELECT:

· SO Loss of Control Room at Taylor, OR;

· SO Loss of Control Room at Bastrop.

An email will be received after 10 minutes with a report of who has responded along with their estimated time of arrival.

	QSE

Hotline

Call
	Q#106 - Typical Hotline Script Emergency Notice Loss of Primary Control Center (Systems are Functional) 



	Additional Notifications
	The Director Control Room Operations and/or Designee will notify the following:

· Service Desk (to notify GMS Support and IT Infrastructure (Telecommunications and Data Center)

· Engineering Support

These phone numbers are also programmed into the control room cell phone.

	ERCOT Website
Posting
	As time permits, post the Emergency Notice on the ERCOT Website.

Typical Posting Script:
ERCOT has issued an Emergency Notice for the loss of the primary control center, all systems are functioning.

	Respond to QSEs
	Monitor frequency and re-run SCED/use (manual offset) as needed.

IF:

· QSEs call in with questions about operational timelines or non real-time issues;

THEN:

· Notify the QSE that timelines and non real-time issues will be addressed when additional staff arrive,

· Document QSE calls to pass along to the appropriate Operators as they arrive.

	Real-time Contingency Analysis (RTCA)
	Operations Support will assist in monitoring the RTCA functionality including results and may communicate suggested actions as needed.

	DC Tie E-Tags
	Make every attempt to handle all DC Tie E-Tags.

If the workload makes it impossible to keep up with, the E-Tags will be passively denied.

	Log
	Log all actions taken.

	Systems are not Functional

	NOTE
	Market Participants must be notified of unplanned outages of 30 minutes or more, of telemetering, monitoring and assessment capabilities, and associated communication channels between affected entities.

	Monitor Frequency
	The ability to view an adequate Frequency source may be limited during a site-failover, database load, or if AGC is temporarily unavailable.  To view the System Frequency during these conditions you may monitor the following sources.

· ERCOT control room digital wall frequency displays

· PI ProcessBook → ERCOT → TrueTime Frequency (Taylor) and/or

· PI ProcessBook → ERCOT → TrueTime Frequency (Bastrop)
It may be necessary to reload the PI ProcessBook “ERCOT Main Summary” display to show the historical data.

	Constant

Frequency
	IF:

· The loss of PCC involves the loss of EMS (LFC and RLC);

THEN:

· Instruct a QSE to go on Constant Frequency

· Q#114 – Typical Script to Instruct a QSE on Constant Frequency for LRC/SCED Failure

· Refer to Real-time desk procedure section 3.3, EMMS (LFC and RLC/SCED) Failure 



	Call in Additional Operators
	Coordinate with the Transmission and Security Desk Operator and activate the NXT scenario to call in additional shift personnel, which includes a Shift Supervisor. 

 SELECT:

· SO Loss of Control Room at Taylor, OR;

· SO Loss of Control Room at Bastrop.

An email will be received after 10 minutes with a report of who has responded along with their estimated time of arrival.

	QSE

Hotline

Call
	Q#107 - Typical Hotline Script Emergency Notice Loss of Primary Control Center (Systems are NOT Functional) 

If either Hotlines are inoperable, phone numbers can be found on SharePoint under OPX & LD Numbers.

	ERCOT Website
Posting
	As time permits, post the Emergency Notice on the ERCOT Website.

Typical Posting Script:
ERCOT has issued an Emergency Notice for the loss of the primary control center, systems are not functioning.

	Additional Notifications
	The Director Control Room Operations and/or Designee will notify the following:

· Service Desk (to notify GMS Support and IT Infrastructure (Telecommunications and Data Center)

· Engineering Support

These phone numbers are also programmed into the control room cell phone.

	Respond to QSEs
	IF:

· The QSE that is put on constant frequency needs assistance controlling frequency;

THEN:

· Issue a unit specific Operating Instruction to another QSE to help with Regulation.  (Follow-up with electronic Dispatch Instruction as time permits)

OR
· Put a different QSE on constant frequency (Remember to take the first QSE off constant frequency).

	Real-time Contingency Analysis (RTCA)
	Operations Support will be responsible for the State Estimator and RTCA functionality and ensuring an RTA is performed at least every 30 minutes.  They will communicate results and any suggested actions as needed.

	DC Tie Tags
	Make every attempt to handle all DC Tie E-Tags.

If the workload makes it impossible to keep up with, the E-Tags will be passively denied.

	Electronic

Dispatch 

Instruction
	Follow-up with electronic Dispatch Instructions where appropriate when systems are functional or as time permits (See Desktop Guide Common to Multiple Desks section 2.3).

When issuing a VDI or confirmation, ensure the use of three-part communication:

· Issue the Operating Instruction

· Receive a correct repeat back

· Give an acknowledgement



	Log
	Log all actions taken.


4.5
Restoration of Primary Control Center Functionality

Procedure Purpose: To be performed once additional staff has reported to the alternate Control Center and Grid can return to normal operations.

	Protocol Reference
	
	
	
	

	Guide Reference
	
	
	
	

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	NOTE:
	As additional Operators arrive, communicate any pertinent information that will assist them in getting their specific functions ready for normal operation.

	Additional

Operators
	When a Real-Time Desk Operator has arrived, turn over frequency control to that Operator and assist with other needs such as approve any E-Tags waiting.

	Notification of Additional Operators
	WHEN:

· Fully staffed;

THEN:

· Coordinate with the Control room and make notification using the NXT - SO Loss of CC Operations Resumed.

	Help

Desk
	Make notification to Service Desk that the Control Room is now back to normal operations from the alternate.

	Contact Security
	Notify Security that the transition of Operations to the ACC has been completed.

	Log
	Make log entries as needed.


4.6
Market Notifications

Procedure Purpose: Guidelines for issuing Emergency Conditions and the four possible levels: Operating Condition Notices (OCN), Advisories, Watches, and Emergency Notices.

	Protocol Reference


	6.3.3
	6.3.3
	6.5.9.3.1
	6.5.9.3.2

	
	6.5.9.3.3
	6.5.9.3.4
	
	

	Guide Reference
	4.2.1
	4.2.2
	4.2.3
	4.2.4

	NERC Standard
	
	
	
	


	Version: 2 
	Revision: 0
	Effective Date:  December 5, 2025


	Step
	Action

	NOTE
	· ERCOT is in an Emergency Condition whenever ERCOT Transmission Grid status is such that a violation of security criteria presents the threat of uncontrolled separation or cascading Outages and/or large-scale service disruption to Load (other than Load being served from a radial transmission line) and/or overload of a Transmission Element, and no timely solution is obtainable through SCED or CMPs.

· Consider the severity of the potential Emergency Condition prior to the issuance of a notification.  The severity of the Emergency Condition could be limited to an isolated local area, or the condition might cover large areas affecting several entities, or the condition might be an ERCOT-wide condition potentially affecting the entire ERCOT System.

· The sequence of notifications issued may vary due to changing system conditions or other operational issues and it may be necessary to skip certain notifications due to the severity of the situation.

	Operating Condition Notice (OCN)

	NOTE
	OCN’s are used to inform Market Participants of a possible future need for more Resources due to conditions that could affect ERCOT System reliability.  OCNs are for informational purposes only and may solicit additional information to determine whether the issuance of an Advisory, Watch, or Emergency Notice is warranted. OCNs serve as a reminder to QSEs and TSPs that some attention to the changing conditions may be warranted.

	1
	As directed by the Shift Supervisor or when appropriate, issue an OCN.  The OCN can be issued for any of the following reasons:

· Insufficient Resources to meet forecasted condition

· There is a projected reserve capacity shortage In DRUC that could affect reliability and may require more Resources

· When extreme cold weather is developing and forecasted to impact the ERCOT Region
· When extreme hot weather is forecasted to impact the ERCOT Region

· When a Hurricane or Tropical Storm is developing and forecasted to impact the ERCOT Region  

· Unplanned transmission Outages that may impact System reliability
When anticipated adverse weather conditions are forecasted, ERCOT may confer with TOs and QSEs regarding the potential for adverse reliability impacts and contingency preparedness.

	Advance Action Notice (AAN)

	NOTE
	Outage Adjustment Evaluation (OAE)

Outage Schedule Adjustment (OSA)

An AAN is a type of OCN, ERCOT may issue an AAN in anticipation of a possible Emergency Condition.  An AAN will identify actions ERCOT expects to take to address the condition unless the need for ERCOT action is alleviated by QSE and/or TSP actions or by other system developments.

	1
	As instructed by the Shift Supervisor and in coordination with Outage Coordination and Operations Support, issue an AAN.  The AAN can be issued for any of the following reasons:

· Insufficient Resources to meet forecasted conditions

· There is a projected reserve capacity shortage in DRUC that could affect reliability and may require more Resources
· When extreme cold weather is developing and forecasted to impact the ERCOT Region
· When extreme hot weather is forecasted to impact the ERCOT Region

· When a Hurricane or Tropical Storm is developing and forecasted to impact the ERCOT Region 

· Unplanned transmission Outages that may impact reliability

When anticipated adverse weather conditions are forecasted, ERCOT may confer with TOs and QSEs regarding the potential for adverse reliability impacts and contingency preparedness.

	Advisory

	1
	As directed by the Shift Supervisor or when appropriate, issue an Advisory.  The Advisory can be issued for any of the following reasons:

· When the probability of an approaching Hurricane or Tropical Storm impacting the ERCOT Region increases, and concerns exit to escalate awareness  

· When the probability of extreme cold weather impacting the ERCOT Region increases, and concerns exist to escalate awareness
· When the probability of extreme hot weather impacting the ERCOT Region increases, and concerns exist to escalate awareness

· When conditions are developing or have changed and more Ancillary Services will be needed to maintain current or near-term reliability

· ERCOT exercises its authority to increase Ancillary Service requirements above the quantities originally specified in the Day-Ahead Market in accordance with ERCOT procedures.

· When extreme weather or ERCOT System conditions require more lead-time than the normal Day-Ahead Market allows.

· Transmission system conditions are such that operations within security criteria are not likely or possible because of Forced Outages or other conditions unless a CMP exists

· Loss of communications or control condition is anticipated or significantly limited

· ERCOT may require information from QSEs representing Resources regarding the Resources’ fuel capabilities. Requests for this type of information shall be for a time period of no more than seven days from the date of the request

	Watch

	1
	As directed by the Shift Supervisor or when appropriate, issue a Watch.  The Watch can be issued for any of the following reasons:

· A reserve capacity shortage is projected with no market solution available that could affect reliability 

· When an approaching Hurricane or Tropical Storm is imminent and anticipated to have an adverse impact on the ERCOT Region

· When impacts from extreme cold weather is imminent and anticipated to have an adverse impact on the ERCOT Region
· When impacts from extreme hot weather is imminent and anticipated to have an adverse impact on the ERCOT Region

· Conditions have developed such that additional Ancillary Services are needed in the Operating Period

· 
· Market-based congestion management techniques embedded in SCED will not be adequate to resolve transmission security violations

· Forced Outages or other abnormal operating conditions have occurred, or may occur that would require ERCOT to operate with active violations of security criteria as defined in the Operating Guides unless a CMP exists

· The SCED process fails to reach a solution, whether or not ERCOT is using one of the measures in Failure of the SCED Process.

· The need to immediately procure Ancillary Services from existing offers

· ERCOT may instruct TOs to reconfigure transmission elements as necessary to improve the reliability of the system
· ERCOT may require information from QSEs representing Resources regarding the Resources’ fuel capabilities. Requests for this type of information shall be for a time period of no more than seven days from the date of the request

	Emergency Notice

	1
	As directed by the Shift Supervisor or when appropriate, issue an Emergency Notice.  The Emergency Notice can be issued for any of the following reasons:

· Loss of Primary Control Center functionality

· Load Resource deployment for North-Houston voltage stability

· ERCOT cannot maintain minimum reliability standards (for reasons including fuel shortages) during the Operating Period using every Resource practically obtainable from the market

· Immediate action cannot be taken to avoid or relieve a Transmission Element operating above its Emergency Rating

· ERCOT forecasts an inability to meet applicable Reliability Standards and it has exercised all other reasonable options

· A transmission condition has been identified that requires emergency energy from any of the DC Ties or curtailment of schedules

· The Transmission Grid is such that a violation of security criteria as defined in the Operating Guides presents the threat of uncontrolled separation or cascading outages, large-scale service disruption to load (other than Load being served from a radial transmission line) and/or overload of  Transmission Elements and no timely solution is obtainable through SCED or CMPs

· When extreme cold weather is beginning to have an adverse impact on the ERCOT System
· When extreme hot weather is beginning to have an adverse impact on the ERCOT System
· When a Hurricane or Tropical Storm is beginning to have an adverse impact on the System

	Notifications

	HOTLINE
	Notify QSEs of Notice

Typical script:
“This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back. This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is issuing a [state Notice type] for [state reason].  [QSE] please repeat this back to me.  That is correct, thank you.”

Notify TOs of Notice

Typical script:
“This call requires everyone to remain on the line until it is complete.  [TO] I will be asking you for the repeat back. This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is issuing a [state Notice type] for [state reason].  [TO] please repeat this back to me.  That is correct, thank you.”

All Operating Instructions shall be in a clear, concise, and definitive manner.  Ensure the recipient of the instruction repeats the information back correctly.  Acknowledge the response as correct or repeat the original statement to resolve any misunderstandings.

	POST
	· All notices must be posted on the ERCOT Website using Grid Conditions Communications (GCC) Notices. 

· For “free form” messages, the only option is “public”

· For “free form” messages, the “Notice priority” will be specified as follows:

· Operational Information/OCN type messages – low priority

· Advisory/Watch type messages – medium priority

· Emergency type messages – high priority

	Hotline Cancellation
	Typical Hotline Script cancellation:
 “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back. This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is cancelling the [state Notice type] for [state reason].  [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

	LOG
	Make log entry.


Document Control

Manual Change History

	Procedure
	Ver/Rev
	Reason for Issue
	Effective Date

	All Sections
	2.0 / 0
	RTC+B Implementation 

All procedures in this manual have been reviewed
	December 5, 2025



