	ERCOT Operating Procedure Manual

	Resource Desk


	ERCOT Operating Procedure Manual

	Resource Desk



Power Operations Bulletin # 2014
ERCOT has posted/revised the Resource Desk manual.

The Various Changes are shown below.

A copy of the procedure can be found at:

http://www.ercot.com/mktrules/guides/procedures/index.html
3.6
Deploying Non-Spin for Local Congestion

Procedure Purpose:  Mitigating local congestion.

	Protocol Reference
	6.5.7.6.2.3(1)
	
	
	

	Guide Reference 
	
	
	
	

	NERC Standard
	EOP-011-2

R1, R1.1, R1.2, R1.2.4
	IRO-001-4

R1
	TOP-001-6

R1
	


	Version: 2 
	Revision: 1
	Effective Date:  January 30, 2026


	Step
	Action

	1
	IF:

· Notified by the ERCOT Transmission and Security Desk Operator that specific Resources awarded Non-Spin are needed for local congestion; 

THEN:

· Deploy specific Resources OR

· Use ASM Non-Spin Deployment for Transmission Constraints (Refer to Desktop Guide Resource Desk 2.1.3.1)

	Note
	If Non-Spin is deployed from one operating day to the next operating day, new deployments will need to be sent for the new day.

	2
	IF:

· The Non-Spin obligation ends for the Resource deployed, AND

· The Resource is still needed; 

THEN:

· Request the RUC Desk Operator to issue an electronic Dispatch Instruction to RUC commit

	3
	IF:

· An additional Resource is needed to be deployed; 

THEN:

· The previous Resource deployed will need to be deployed again, along with the additional Resource.

	Termination
	IF:
· Notified by the Transmission and Security Desk Operator that Non-Spin is no longer needed;

THEN:
· Recall the Non-Spin, by sending a Zero MW XML (Refer to Desktop Guide Resource Desk Section 2.1.3.2 and 2.1.4).

	Log
	Log all actions.


Document Control

Manual Change History

	Procedure
	Ver/Rev
	Reason for Issue
	Effective Date

	All Sections
	2.0 / 0
	RTC+B Implementation 

All procedures in this manual have been reviewed
	December 5, 2025

	3.6
	2.0 / 1
	Added Transmission Constraint Deployment
	January 30, 2026



