	ERCOT Operating Procedure Manual

	Scripts



Power Operations Bulletin # 2023
ERCOT has posted/revised the Scripts manual.

The Various Changes are shown below.

A copy of the procedure can be found at:

http://www.ercot.com/mktrules/guides/procedures/index.html 

Q#116 Start Constant Frequency Control Test:
“This is ERCOT Operator [first and last name]. At [time], ERCOT is conducting an unannounced constant frequency control test with [QSE].  The frequency bias to enter into your control system is [bias].  The test will be up to 15 minutes. I will follow up with an electronic Dispatch Instruction confirmation that you will need to acknowledge. I will give another call when you can return to normal operation.  [QSE] please repeat this back to me.”
If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

Q#117 End Constant Frequency Control Test:

“This is ERCOT Operator [first and last name]. At [time], ERCOT is ending the unannounced constant frequency control test with [QSE]; please remove the bias and change your control mode back to normal. I will follow-up with an electronic Dispatch Instruction confirmation that you will need to acknowledge.  [QSE] please repeat this back to me.”

If repeat back is CORRECT, “That is correct, thank you.”

If INCORRECT, repeat the process until the repeat back is correct.

Document Control

Manual Change History
	Procedure
	Ver/Rev
	Reason for Issue
	Effective Date

	All Sections
	2.0 / 0
	RTC+B
	December 5, 2025

	2
	2.0 / 1
	Updated Q#126 AAN
	January 1, 2026

	3
	2.0 / 2
	Updated Q#116 Start CFC Test

Updated Q#117 End CFC Test
	April 1, 2026


